*Lecture Experiments of “Embedded Programming”:-

01. Led Blinking:-

void setup() {

volatile char *dirf = 0x30; *dirf=0xff;

volatile char *outf = 0x31;

volatile long i;

while(1)

{

*outf=0x01;

for(i=0; i<100000; i++);

*outf=0x00;

for(i=0; i<100000; i++);

}

}

void loop() {}

02. Led Scrooling:-

#include "dk.h"

void setup() {

initf(0xff);

volatile long i, j;

volatile char x;

while(1)

{

for(j=1; j<8; j=j+2)

{

for(x=0; x<8; x++){

outputf(j << x);

for(i=0; i<100000; i++);

}

for(x=8; x>0; x--){

outputf(j << x);

for(i=0; i<100000; i++);

}

}

}

}

void loop() {}

03. Led & Switch:-

void setup() {

volatile char *dirf, *dirk, *outf, *ink;

dirf=0x30; *dirf=0x01; // port f set as output

dirk=0x107; *dirk=0x00; // port k set as input

outf=0x31;

ink=0x106;

volatile char x;

while(1){

x=*ink;

if((x & 1)==1) *outf=1;

else *outf=0;

}

}

void loop() {}

04.Two Leds & Switch:-

void setup() {

volatile char *dirf, *dirk, *outf, *ink;

dirf=0x30; *dirf=0x03; // pin 1 & 2 of port f set as output

dirk=0x107; *dirk=0x00; // port k set as input

outf=0x31;

ink=0x106;

volatile char x, i=0;

volatile long j;

while(1){

x=*ink;

if((x & 1)==1) i++;

for(j=0; j<50000; j++);

if(i%2) *outf=1;

else *outf=2;

}

}

void loop() {}

05. Seven Segment:-

void setup() {

volatile char *dirf, *outf;

volatile char a[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,0x07,0x7f, 0x6f};

dirf=0x30;

*dirf=0xff;

outf=0x31;

volatile long i, j;

while(1) {

for(i=0; i<10; i++)

{

*outf=a[i];

for(j=0; j<500000; j++);

}

}

}

void loop() {}

06. Keypad:-

void setup() {

volatile char *dirf, *outf, *dirk, *ink, *dira, *outa, *dirc, *outc;

volatile long i, j;

dirf=0x30; *dirf=0xff;     outf=0x31;

dirk=0x107; *dirk=0x00;    ink=0x106;

dira=0x21; *dira=0xff;     outa=0x22;

dirc=0x27; *dirc=0xff;     outc=0x28;

while(1){

for(i=0; i<4; i++)

{

*outf = 1 << i;

*outa = *ink;

if(*ink != 0){

*outc = 1 << i;

for(j=0; j<500000; j++);

*outc = 0;

}

}

}

}

void loop() {}

07. 8*8 Dot Matrix:-

void setup() {

volatile char *dirf, *dirk, *outf, *outk;

volatile long k;

char row, col;

dirf=0x30; *dirf=0xff;

dirk=0x107; *dirk=0xff;

outf=0x31; outk=0x108;

while(1)

{

for(row=0; row<8; row++){

*outk= ~ (1 << row);

for(col=0; col<8; col++){

*outf= 1 << col;

for(k=0; k<100000; k++);

}

*outk= 0xff;

*outf= 0;

}

}

}

void loop() {}

08. 8*8 Dot Matrix Display [ECEN]:-

void setup() {

volatile char *dirf, *dirk, *outf, *outk;

volatile long a;

char i, count=0;

dirf=0x30; *dirf=0xff;

dirk=0x107; *dirk=0xff;

outf=0x31; outk=0x108;

volatile char ecen[][8] = {

{0b11111111,

0b11111111,

0b11000000,

0b11111111,

0b11111111,

0b11000000,

0b11111111,

0b11111111},

{0b00111110,

0b01111111,

0b11100011,

0b11000000,

0b11000000,

0b11100011,

0b01111111,

0b00111110},

{0b11111111,

0b11111111,

0b11000000,

0b11111111,

0b11111111,

0b11000000,

0b11111111,

0b11111111},

{0b11000011,

0b11100011,

0b11110011,

0b11111011,

0b11011111,

0b11001111,

0b11000111,

0b11000011}};

while(1){

for(a=0; a<20000; a++){

for(i=0; i<8; i++)

{

*outk= ~(1 << i);

*outf = ecen[count][i];

*outk=0xff;

*outf=0;

}

}

for(a=0; a<45000; a++);

count++;

if(count==4) count=0;

}

}

void loop() {}

* Embedded Programming Problem Sets:-

| 1. Glow LSB LED only (bit 0):- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=1; } void loop() {} |  |
| --- | --- |
| 2. Glow MSB LED only (bit 7):- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0x80; } void loop() {} | 3. Glow LSB 2 LEDs only (bit 0 & bit 1):- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=3; } void loop() {} |
| 4. Glow MSB 2 LEDs only (bit 7 & bit 6):- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0xc0; } void loop() {} | 5. Glow LSB 4 LEDs only (bit 0 to bit 3):- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0x0f; } void loop() {} |

ESP - LED Problems – Set 1

| 6. Glow MSB 4 LEDs only (bit 7 to bit 4):- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0xf0; } void loop() {} |  |
| --- | --- |
| 7. Glow all 8 LEDs:- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0xff; } void loop() {} | 8. Glow all ODD no LEDs:- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0xaa; } void loop() {} |
| 9. Glow all EVEN no LEDs:- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0x55; } void loop() {} | 10. Glow LEDs of bit7, bit4, bit3, bit1:- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0x99; } void loop() {} |

ESP - LED Problems – Set 2

| 1. Blink First LED (With Delay):- void setup() { volatile char *dirf, *outf; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { *outf=1; for(i=0; i<100000; i++); *outf=0; for(i=0; i<100000; i++); } } void loop() {} |  |
| --- | --- |
| 2. Blink all the 8 LEDs (with delay):- void setup() { volatile char *dirf, *outf; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { *outf=0xff; for(i=0; i<100000; i++); *outf=0; for(i=0; i<100000; i++); } } void loop() {} | 3. Glow the LEDs one by one (Do not off the LED):- void setup() { volatile char *dirf, *outf, a; volatile long i; volatile short arr[]={2, 4, 8, 16, 32, 64, 128, 256}; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<8; a++) { *outf=arr[a]-1; for(i=0; i<100000; i++); } *outf=0; } } void loop() {} |

| 4. Glow the LEDs one by one(at a time one LED should glow):- void setup() { volatile char *dirf, *outf, a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<8; a++) { *outf=1 << a; for(i=0; i<500000; i++); } } } void loop() {} | 5. Glow the EVEN LEDs one by one (Do not off the LED):- void setup() { volatile char *dirf, *outf, a; volatile long i; volatile char arr[]={1, 4, 16, 64}; volatile short all=0; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { all += arr[a]; *outf=all; for(i=0; i<100000; i++); } if(all >= 85) all=0; *outf=0; } } void loop() {} |
| --- | --- |
| 6. Glow the ODD LEDs one by one (Do not off the LED):- void setup() { volatile char *dirf, *outf, a; volatile long i; volatile char arr[]={1, 4, 16, 64}; volatile short all=0; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { all += (arr[a] * 2); *outf=all; for(i=0; i<100000; i++); } if(all >= (85 * 2)) all=0; *outf=0; } } void loop() {} | 7. Glow 0 to 3 LEDs (one by one):- void setup() { volatile char *dirf, *outf, a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { *outf= 1 << a; for(i=0; i<100000; i++); } } } void loop() {} |

| 8. Glow 4 to 7 LEDs (one by one):- void setup() { volatile char *dirf, *outf, a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { *outf= 16 << a; for(i=0; i<100000; i++); } } } void loop() {} |  |
| --- | --- |
| 9. Glow 3 to 0 LEDs (one by one):- void setup() { volatile char *dirf, *outf, a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { *outf= 8 >> a; for(i=0; i<100000; i++); } } } void loop() {} | 10. Glow 7 to 4 LEDs (one by one):- void setup() { volatile char *dirf, *outf, a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { *outf= 128 >> a; for(i=0; i<100000; i++); } } } void loop() {} |

ESP - LED Problems – Set 3

| 1. Glow the LEDs in following order 3,7 delay – 2,6 delay – 1,5 delay – 0,4 delay:- void setup() { volatile char *dirf, *outf, a; volatile char arr[]={136, 68, 34,17}; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} |  |
| --- | --- |
| 2. Glow the LEDs in following order 0,4 delay – 1,5 delay – 2,6 delay – 3,7 delay:- void setup() { volatile char *dirf, *outf, a; volatile char arr[]={136, 68, 34,17}; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=3; a>=0; a--) { *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} | 3. Glow 0 to 7 LEDs with delay and 7 to 0 LEDs with delay:- void setup() { volatile char *dirf, *outf, a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1){ for(a=0; a<8; a++) { *outf= 1<<a; for(i=0; i<100000; i++); } for(a=0; a<8; a++) { *outf= 128>>a; for(i=0; i<100000; i++); } } } void loop() {} |

| 4. Glow 0,2,1,3,4,6,5,7 LEDs with delay:- void setup() { volatile char *dirf, *outf, a, count=0; volatile long i; volatile char arr[]={1,4,2,8,16,64,32,128}; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<8; a++) { *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} | 5. Glow the LEDs in following order 0,1 delay – 6,7, delay – 2,3, delay – 4,5, delay void setup() { volatile char *dirf, *outf, a, count=0; volatile long i; volatile char arr[]={3,192,12,48}; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} |
| --- | --- |
| 6. Glow the LEDs in following order 4,5 delay – 2,3, delay – 6,7, delay – 0,1, delay:- void setup() { volatile char *dirf, *outf, a, count=0; volatile long i; volatile char arr[]={3,192,12,48}; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=3; a>=0; a--) { *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} | 7. Glow the LEDs in following order 0,1,2 delay – 5,6,7, delay – 3,4, delay:- void setup() { volatile char *dirf, *outf, a, count=0; volatile long i; volatile char arr[]={7,224,24}; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<3; a++) { *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} |

| 8. Glow the LEDs in following order 3,4 delay – 5,6,7, delay – 0,1,2, delay:- void setup() { volatile char *dirf, *outf, a, count=0; volatile long i; volatile char arr[]={7,224,24}; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=3; a>=0; a--) { *outf=arr[a]; for(i=0; i<200000; i++); } } } void loop() {} |  |
| --- | --- |
| 9. Glow the LEDs in following order 0 then 0,1 then 0,1, 2…... then 0,1,2,3,4,5,6,7 with delay.:- void setup() { volatile char *dirf, *outf, a, count=0; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<8; a++) { *outf=(2 << a ) -1; for(i=0; i<200000; i++); } } } void loop() {} | 10. Glow the LEDs in following order 7 then 7,6 then 7,6, 5…... then 7,6,5,4,3,2,1,0 with delay.:- void setup() { volatile char *dirf, *outf, a, count=0; volatile long i; volatile char arr[]={128,192,224,240,248,252,254,255}; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<8; a++) { *outf=arr[a]; for(i=0; i<200000; i++); } } } void loop() {} |

ESP - Switch Problems – Set 1

| 1. Press 0th switch glow 8 LEDs:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile char x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x==1){ *outf=0xff; for(i=0; i<100000; i++); } } } void loop() {} |  |
| --- | --- |
| 2. Press 2nd switch blink 8 LEDs:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile char x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0; outf=0x31; ina=0x20; while(1){ x=*ina; if(x==4){ while(1){ *outf=0xff; for(i=0; i<100000; i++); *outf=0; for(i=0; i<100000; i++); } } } } void loop() {} | 3. Press 3rd switch glow 0th LED, delay,1st LED, delay …:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile char x, a; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x==8){ for(a=0; a<8; a++){ *outf= 1 << a; for(i=0; i<100000; i++); } } } } void loop() {} |

| 4. Press any switch glow 7th LED, delay,6th LED, delay …:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile char x, a; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x){ for(a=0; a<8; a++){ *outf= 128 >> a; for(i=0; i<100000; i++); } } } } void loop() {} | 5. Press 0th switch glow even LEDs:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile char x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x==1){ *outf=0x55; for(i=0; i<500000; i++); } } } void loop() {} |
| --- | --- |
| 6. Press 7th switch glow odd LEDs:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile short x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if((x & 128) == 128){ *outf=0xaa; for(i=0; i<500000; i++); } } } void loop() {} | 7. Press 0th switch 0th LED should glow Press 1st switch 1st LED should glow …....up to 7th switch:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile long x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=*ina; } } void loop() {} |

| 8. Press 0th switch 7th LED should glow …. Press 1st switch 6th LED should glow ….. ...........up to 7th switch:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile short x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ x=*ina; if(x==1) *outf=128; else if(x==2) *outf=64; else if(x==4) *outf=32; else if(x==8) *outf=16; else if(x==16) *outf=8; else if(x==32) *outf=4; else if(x==64) *outf=2; else if((x & 128)==128) *outf=1; else *outf=0; } } void loop() {} |  |
| --- | --- |
| 9. Press any switch 0th LED ON, OFF 1st LED ON, OFF….:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile char a, x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x) for(a=0; a<8; a++){ *outf= 1 << a; for(i=0; i<100000; i++); } } } void loop() {} | 10. Press 1st switch At a time glow 0th LED,3rd LED, 4th LED,7th LED:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile long i; volatile char a, x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x){ *outf=153; for(i=0; i<500000; i++); } } } void loop() {} |

ESP - Switch Problems – Set 2

| 1. Press 2nd switch first glow 1st LED, delay, 2nd LED, delay, 5th LED, delay,6th LED:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile char arr[]={2,4,32,64}, a; volatile unsigned short x; volatile long i; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x==4) for(a=0; a<4; a++){ *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} |  |
| --- | --- |
| 2. Press 3rd switch first glow 3rd LED, delay, 1st LED, delay, 7th LED, delay,4th LED:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile char arr[]={8,1,128,16}, a; volatile unsigned short x; volatile long i; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x==8) for(a=0; a<4; a++){ *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} | 3. Press 2nd switch glow 0th LED, delay, 1 st LED, delay, 5th LED, delay,6th LED:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile char arr[]={1,2,32,64}, a; volatile unsigned short x; volatile long i; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x==4) for(a=0; a<4; a++){ *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} |

| 4. Press 7th switch first glow 0th LED, delay, 1st LED, Delay, 4th LED, delay,7th LED:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile char arr[]={1,2,16,128}, a; volatile unsigned short x; volatile long i; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if((x & 128)==128) for(a=0; a<4; a++){ *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} | 5. Press 3rd switch first glow 7th LED, delay, 6th LED, delay, 3rd LED, delay,2nd LED:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile char arr[]={128,64,8,4}, a; volatile unsigned short x; volatile long i; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x ==8) for(a=0; a<4; a++){ *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} |
| --- | --- |
| 6. Press 2nd switch first glow 3rd LED, delay, 5th LED, delay, 7th LED, delay,6th LED:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile char arr[]={8,32,128,64}, a; volatile unsigned short x; volatile long i; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x ==4) for(a=0; a<4; a++){ *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} | 7. Press 7th switch first glow 5th LED, delay, 3rd LED, delay, 0th LED, delay,1st LED:- void setup() { volatile char *dirf, *outf, *dira, *ina; volatile char arr[]={32,8,1,2}, a; volatile unsigned short x; volatile long i; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if((x & 128) ==128) for(a=0; a<4; a++){ *outf=arr[a]; for(i=0; i<500000; i++); } } } void loop() {} |

| 8. Press 1st switch & Press 7th switch Together, Glow 5th LED for 30 seconds:- void delays(char y); void setup() { volatile char *dirf, *outf, *dira, *ina; volatile unsigned short x; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; while(1){ *outf=0; x=*ina; if(x==130){ *outf=32; delays(30); } } } void loop() {} void delays(char y){ // delay(10); = 10 second delay; volatile long i, j; for(i=0; i<y; i++) for(j=0; j<430000; j++); } |  |
| --- | --- |
| 9. Press 1st switch & Press 4th switch & Press 6th switch & Press 2nd switch One by One, Glow 7th LED for 10 seconds:- void delays(char y); void setup() { volatile char *dirf, *outf, *dira, *ina; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; L1: while(1) if(*ina==2) while(1) if(*ina==16) while(1) if(*ina==64) while(1) if(*ina==4) { *outf=128; delays(10); *outf=0; goto L1; } } void loop() {} void delays(char y){ // delay(10); = 10 second delay; volatile long i, j; for(i=0; i<y; i++) for(j=0; j<430000; j++); } | 10. Press 1st switch & Press 4th switch & Press 6th switch & Press 5th switch One by One, Glow 5th LED; Press 4th switch & Press 6th switch & Press 5th switch & Press 3th switch & One by One, OFF 5th LED:- void delays(char y); void setup() { volatile char *dirf, *outf, *dira, *ina; dirf=0x30; *dirf=0xff; dira=0x21; *dira=0x00; outf=0x31; ina=0x20; L1: while(1) { if(*ina==2){ while(1) if(*ina==16) while(1) if(*ina==64) while(1) if(*ina==32) {*outf=32; goto L1;}} if(*ina==16){ while(1) if(*ina==64) while(1) if(*ina==32) while(1) if(*ina==8) {*outf=0; goto L1;}} } } void loop() {} |

ESP – 7 Segment Problems – Set 1

| 01 Display 0 to 9 without decimal points:- void setup() { volatile char *dirf, *outf; volatile char a[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; volatile long i, j; while(1) { for(i=0; i<10; i++) { *outf=a[i]; for(j=0; j<300000; j++); } } } void loop() {} |  |
| --- | --- |
| 02 Display 0 to 9 with decimal points:- void setup() { volatile char *dirf, *outf; volatile char a[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; volatile long i, j; while(1) { for(i=0; i<10; i++) { *outf=a[i] + 128; for(j=0; j<300000; j++); } } } void loop() {} | 03 Display 9 to 0 with delay:- void setup() { volatile char *dirf, *outf; volatile char a[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; volatile long i, j; while(1) { for(i=9; i>=0; i--) { *outf=a[i]; for(j=0; j<300000; j++); } } } void loop() {} |

| 04 Display odd numbers with delay:- void setup() { volatile char *dirf, *outf; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; volatile char a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=1; a<10; a+=2) { *outf=arr[a]; for(i=0; i<300000; i++); } } } void loop() {} | 05 Display even numbers with delay:- void setup() { volatile char *dirf, *outf; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; volatile char a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<10; a+=2) { *outf=arr[a]; for(i=0; i<300000; i++); } } } void loop() {} |
| --- | --- |
| 06 Display the possible alphabets ( A to Z):- void setup() { volatile char *dirf, *outf; volatile char arr[]={0x77, 0x7c,0x58,0x5e, 0x79,0x71,0x40,0x76,0x06,0x0e,0x40,0x38, 0x40,0x80,0x5c,0x73,0x67,0x50,0x6d,0x40, 0x1c,0x40,0x80,0x40,0x80,0x40}; volatile char a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; for(a=0; a<26; a++) { *outf=arr[a]; for(i=0; i<500000; i++); } *outf=0; } void loop() {} | 07 Display numbers in following order: 0,2,4,6,8,9,7,5,3,1:- void setup() { volatile char *dirf, *outf; volatile char arr[]={0x3f,0x5b,0x66,0x7d,0x7f, 0x6f,0x07,0x6d,0x4f, 0x06}; volatile char a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<10; a++) { *outf=arr[a]; for(i=0; i<300000; i++); } } } void loop() {} |

| 08 Display the single digit prime numbers:- void setup() { volatile char *dirf, *outf; volatile char arr[]={0x5b,0x4f,0x6d,0x07}; volatile long i; volatile char a; dirf=0x30; *dirf=0xff; outf=0x31; while(1) { for(a=0; a<4; a++) { *outf=arr[a]; for(i=0; i<300000; i++); } } } void loop() {} |  |
| --- | --- |
| 09 Display last digit of a number. Assume any number.:- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0x6d; } void loop() {} | 10 Display first digit of a number. Assume any number:- void setup() { volatile char *dirf, *outf; dirf=0x30; *dirf=0xff; outf=0x31; *outf=0x5b; } void loop() {} |

ESP – 7 Segment Problems – Set 2

| 01 Press any switch & display that number. Ex: 1 - > 1, 2 ->2, … 8 -> 8:- void delays(char y); void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char arr[]={0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f}; volatile char a; volatile unsigned short x; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; while(1) { x=*ink; if(x) for(a=0; a<8; a++) { *outf=arr[a]; delays(1); *outf=0x40; delays(1); *outf=arr[a]; delays(1); } *out=0; } } void loop() {} void delays(char y){ // delay(10); = 10 second delay; volatile long i, j; for(i=0; i<y; i++) for(j=0; j<430000; j++); } | 02 Press any switch & display the reverse order Ex: 1 -> 8, 2-> 7, 8 -> 1:- void delays(char y); void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char arr[]={0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f}; volatile char a, b; volatile unsigned short x; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; while(1) { x=*ink; if(x) for(a=0, b=7; a<8; a++, b--){ *outf=arr[a]; delays(1); *outf=0x40; delays(1); *outf=arr[b]; delays(1); } *outf=0; } } void loop() {} void delays(char y){ // delay(10); = 10 second delay; volatile long i, j; for(i=0; i<y; i++) for(j=0; j<430000; j++); } |
| --- | --- |

| 03 Press any switch & Display should start from that number till nine and restart from that number till 9. Continuous loop:- void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; volatile char a, b; volatile long i; volatile unsigned short x; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; while(1) { x=*ink; if(x==1) a=0; else if(x==1) a=0; else if(x==2) a=1; else if(x==4) a=2; else if(x==8) a=3; else if(x==16) a=4; else if(x==32) a=5; else if(x==64) a=6; else if((x & 128)==128) a=7; if(x) while(1){ for(b=a; b<10; b++){ *outf=arr[b]; for(i=0; i<200000; i++); } } } } void loop() {} | 04 Press switch 7 & display from 0 to 9 and 9 to 0 one time:- void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,0x07,0x7f, 0x6f}; volatile char a; volatile long i; volatile unsigned short x; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; while(1) { x=*ink; *outf=0; if((x & 128)==128){ for(a=0; a<10; a++){ *outf=arr[a]; for(i=0; i<300000; i++); } for(a--; a>=0; a--){ *outf=arr[a]; for(i=0; i<300000; i++); } } } } void loop() {} |
| --- | --- |
| 05 Display A – F in a loop:- void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char arr[]={0x77, 0x7c,0x58,0x5e,0x79,0x71}; volatile char a; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; // dirk=0x107; *dirk=0; ink=0x106; while(1) { for(a=0; a<6; a++){ *outf=arr[a]; for(i=0; i<300000; i++); } } } void loop() {} | 06 Initially display 0. When switch 1 is pressed every time increment by 1:- void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; volatile char count=0; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; *outf=arr[count]; while(1) { if(*ink==2){ count++; *outf=arr[count]; for(i=0; i<100000; i++); } if(count==9) {count=0; count--;} } } void loop() {} |

| 07 Initially display 9. When switch 2 is pressed every time decrement by 1:- void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; volatile char count=9; volatile long i; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; *outf=arr[count]; while(1) { if(*ink==4){ count--; *outf=arr[count]; for(i=0; i<100000; i++); } if(count==0) count=10; } } void loop() {} | 08 Initially display 0 When switch 1 is pressed start displaying 0 to 9 Continuously. When switch 2 is pressed stop display:- void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char a, arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; volatile long i; volatile char flag=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; while(1) { *outf=0x3f; if(*ink==2) while(1){ for(a=0; a<10; a++){ *outf=arr[a]; for(i=0; i<300000; i++) if(*ink==4) flag=1; if(flag==1) break; } if(flag==1){flag=0; break;} } } } void loop() {} |
| --- | --- |
| 09 Initially display 9 When switch 1 is pressed start displaying 9 to 0 Continuously. When Switch 2 is pressed stop display:- void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char a, arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; volatile long i; volatile char flag=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; while(1) { *outf=0x6f; if(*ink==2) while(1){ for(a=9; a>=0; a--){ *outf=arr[a]; for(i=0; i<300000; i++) if(*ink==4) flag=1; if(flag==1) break; } if(flag==1){flag=0; break;} } } } void loop() {} | 10 Get a 4 digit number from user (Press the switch Sequentially (Example 1256) Add the sum of Them the sum is more than 1 digit then add them again and show the sum on display:- void setup() { volatile char *dirf, *outf, *dirk, *ink; volatile char a, arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; volatile char count=0, x, sum=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0; ink=0x106; volatile long i; while(1){ x=*ink; if(x==1) a=0; else if(x==2) a=1; else if(x==4) a=2; else if(x==8) a=3; else if(x==16) a=4; else if(x==32) a=5; else if(x==64) a=6; else if((x & 128)==128) a=7; if(x){ sum+=a; count++; for(i=0; i<90000; i++); } if(count==4) break; } while(sum>9){ sum=(sum/10) + (sum%10); } *outf=arr[sum]; } void loop() {} |

| 01 Display 00 to 99 in loop with delay:- void setup() { volatile char *dirf, *outf, *dirk, *outk; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile long i; volatile char a, b; while(1) { for(a=0; a<10; a++) for(b=0; b<10; b++) { *outf=arr[a]; *outk=arr[b]; for(i=0; i<300000; i++); } } } void loop() {} | Connect TWO 7-Segment Displays to 2 individual ports |
| --- | --- |
| 02 Display 0.0 to 9.9 in loop with delay:- void setup() { volatile char *dirf, *outf, *dirk, *outk; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile long i; volatile char a, b; while(1) { for(a=0; a<10; a++) for(b=0; b<10; b++) { *outf=arr[a] + 128; *outk=arr[b]; for(i=0; i<300000; i++); } } } void loop() {} | 03 Display 99 to 00 in loop with delay:- void setup() { volatile char *dirf, *outf, *dirk, *outk; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile long i; volatile char a, b; while(1) { for(a=9; a>=0; a--) for(b=9; b>=0; b--) { *outf=arr[a]; *outk=arr[b]; for(i=0; i<300000; i++); } } } void loop() {} |

ESP – 7 Segment Problems – Set 3

| 04 Display 9.9 to 0.0 in loop with delay:- void setup() { volatile char *dirf, *outf, *dirk, *outk; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile long i; volatile char a, b; while(1) { for(a=9; a>=0; a--) for(b=9; b>=0; b--) { *outf=arr[a] + 128; *outk=arr[b]; for(i=0; i<300000; i++); } } } void loop() {} | Connect TWO 7-Segment Displays to 2 individual ports |
| --- | --- |
| 05 Display 0000 to 9999 in loop with delay:- void setup() { volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *outc; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0xff; outc=0x28; volatile long i; volatile char a, b, c, d; while(1) { for(a=0; a<10; a++) for(b=0; b<10; b++) for(c=0; c<10; c++) for(d=0; d<10; d++) { *outf=arr[a]; *outk=arr[b]; *outa=arr[c]; *outc=arr[d]; for(i=0; i<50000; i++); } } } void loop() {} | Connect FOUR 7-Segment Displays to 4 individual ports |

| 06 Display 0.000 to 999.9 in loop with delay:- void setup() { volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *outc; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0xff; outc=0x28; volatile long i; volatile char a, b, c, d; volatile char count=0, m, n, o, p; while(1) { if(count==0){m=128; n=0; o=0; p=0;} if(count==1){m=0; n=128; o=0; p=0;} if(count==2){m=0; n=0; o=128; p=0;} if(count==3){m=0; n=0; o=0; p=128;} for(a=0; a<10; a++) for(b=0; b<10; b++) for(c=0; c<10; c++) for(d=0; d<10; d++) { *outf=arr[a] + m; *outk=arr[b] + n; *outa=arr[c] + o; *outc=arr[d] + p; for(i=0; i<1000; i++); } count++; if(count==4) goto end; } end: *outf=0; *outk=0; *outa=0; *outc=0; } void loop() {} | 07 Display 0 to 9999 in loop with delay. Do not display the prefix 0 :- void setup() { volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *outc; volatile char arr[]={0x00, 0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0xff; outc=0x28; volatile long i, count=0; volatile char a, b, c, d; volatile char m, n, o, p; while(1) { for(a=1; a<=10; a++) for(b=1; b<=10; b++) for(c=1; c<=10; c++) for(d=1; d<=10; d++) { if(count>=1000) {m=1; n=1; o=1; p=1;} else if(count>=100) {m=0; n=1; o=1; p=1;} else if(count>=10) {m=0; n=0; o=1; p=1;} else if(count>=1) {m=0; n=0; o=0; p=1;} *outf=arr[a*m]; *outk=arr[b*n]; *outa=arr[c*o]; *outc=arr[d*p]; for(i=0; i<300000; i++); count++; if(count>10000) goto end; } } end: *outf=0; *outk=0; *outa=0; *outc=0; } void loop() {} |
| --- | --- |

| 08 Display 9999 to 0 in loop with delay. Do not display the prefix 0 :- void setup() { volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *outc; volatile char arr[]={0x00, 0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0xff; outc=0x28; volatile long i, count=9999; volatile char a, b, c, d; volatile char m, n, o, p; while(1) { for(a=10; a>0; a--) for(b=10; b>0; b--) for(c=10; c>0; c--) for(d=10; d>0; d--) { if(count<10) {m=0; n=0; o=0; p=1;} else if(count<100) {m=0; n=0; o=1; p=1;} else if(count<1000) {m=0; n=1; o=1; p=1;} else if(count<10000) {m=1; n=1; o=1; p=1;} *outf=arr[a*m]; *outk=arr[b*n]; *outa=arr[c*o]; *outc=arr[d*p]; for(i=0; i<1000; i++); count--; if(count==0) goto end; } } end: *outf=0; *outk=0; *outa=0; *outc=0; } void loop() {} | 09 Display the two-digit prime numbers in loop :- void setup() { volatile char *dirf, *outf, *dirk, *outk;// *dira, *outa, *dirc, *outc; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66, 0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; //dira=0x21; *dira=0xff; outa=0x22; // dirc=0x27; *dirc=0xff; outc=0x28; volatile char a, b, prime, ones, tens; volatile long i; for(a=10; a<100; a++) { for(b=2; b<a; b++) if((a%b)==0) {prime=0; break;} else prime=a; if(prime){ ones=prime%10; tens=prime/10; *outk=arr[ones]; *outf=arr[tens]; for(i=0; i<500000; i++); } } *outf=0; *outk=0; } void loop() {} |
| --- | --- |

10 Display a clock (Min & Sec only) :- void setup() { volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *outc; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,0x07,0x7f, 0x6f}; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0xff; outc=0x28; volatile char a, b, c, d; for(a=0; a<6; a++) for(b=0; b<10; b++) for(c=0; c<6; c++) for(d=0; d<10; d++) { *outf=arr[a]; *outk=arr[b] + 128; *outa=arr[c]; *outc=arr[d]; delay(1000); } *outf=0; *outk=0; *outa=0; *outc=0; } void loop() {}

ESP – Keypad – Problems Set - 1

| 01 Press any switch & display that number. Ex: 1 - > 1, 2 ->2, … 8 -> 8,9 -> 9,0 -> 0 :- volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char checkKey(); void display(volatile char m); void init(); void setup() { volatile char key; init(); while(1){ key=checkKey(); display(key); } } void display(volatile char number){ volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x40}; volatile char a, b, d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; volatile long i; if(number){ for(a=number, b=number; a<(number+11); a++, b++){ if(b==10) b=0; *outk=d4; *outf=arr[b]; for(i=0; i<200000; i++); } } } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key=20; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key=21; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key=22; } else if(row==3){ if(col==1) key=26; else if(col==2) key=25; else if(col==4) key=24; else if(col==8) key=23; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} | 02 Press 4 digits and display that Number. Ex: 1234,6754, 9870, etc :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile short number=0; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if (number==0) number=key; else number = (number*10) + key; number%=10000; thousand=number/1000; hundred=(number%1000)/100; tens=(number%100)/10; ones=number%10; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x40}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} |
| --- | --- |

| 03 Perform Single Digit Addition Ex: 6 + 7 = 13, 9 + 4 = 13 :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile short number=0, num1, num2; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if(key=='+' ¦¦ key=='-' ¦¦ key=='*' ¦¦ key=='/'){ num1=number; number=0;} else if(key=='='){ num2=number; number=num1+num2;} else number=key; if(key=='c') number=num1=num2=0; number%=100; tens=number/10; ones=number%10; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x40}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} | 04 Perform Two Digit Addition Ex: 61 + 72 = 133, 19 + 42 = 61 :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile short number=0, num1, num2; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if(key=='+' ¦¦ key=='-' ¦¦ key=='*' ¦¦ key=='/'){ num1=number; number=0;} else if(key=='='){ num2=number; number=num1+num2;} else if(number==0) number=key; else number = (number*10) + key; if(key=='c') number=num1=num2=0; if(key=='=') number%=1000; else number%=100; hundred=number/100; tens=(number%100)/10; ones=number%10; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x40}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} |
| --- | --- |

| 05 Perform three Digit Addition. Ex: 161 + 656 = 817 659 + 765 = 1424 :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile short number=0, num1, num2; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if(key=='+' ¦¦ key=='-' ¦¦ key=='*' ¦¦ key=='/'){ num1=number; number=0;} else if(key=='='){ num2=number; number=num1+num2;} else if(number==0) number=key; else number = (number*10) + key; if(key=='c') number=num1=num2=0; if(key=='=') number%=10000; else number%=1000; thousand=number/1000; hundred=(number%1000)/100; tens=(number%100)/10; ones=number%10; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x40}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} | 06 Perform any digit Addition. If the total exceeds 9999 then display “EEEE” Ex: 1561 + 2 = 1563, 8765 + 987 = 9752 :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile long number=0, temp, num1, num2; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if(key=='+' ¦¦ key=='-' ¦¦ key=='*' ¦¦ key=='/'){ num1=number; number=0;} else if(key=='='){ num2=number; number=num1+num2;} else if(number==0) number=key; else number = (number*10) + key; if(key=='c') number=num1=num2=0; if(key=='=' && number>9999){thousand=hundred=tens=ones=10; goto level;} temp=number; temp%=10000; thousand=temp/1000; hundred=(temp%1000)/100; tens=(temp%100)/10; ones=temp%10; level:; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x79}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} |
| --- | --- |

| 07 Perform any digit Subtraction. If the total exceeds 9999 then display “EEEE”. Display Negative result also. Ex: 1561 - 2 = 1559, 765 - 987 = -22 :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile long number=0, temp, num1, num2, negative; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if(key=='+' ¦¦ key=='-' ¦¦ key=='*' ¦¦ key=='/'){ num1=number; number=0;} else if(key=='='){ num2=number; number=num1-num2;} else if(number==0) number=key; else number = (number*10) + key; if(key=='c') {number=num1=num2=0; negative=0;} if(key=='=' && number>9999){thousand=hundred=tens=ones=10; goto level;} if(number<0){negative=number; number-=1; number = ~number;} // For -ve value to +ve value conversion temp=number; temp%=10000; if(negative<0 && number<999) thousand=11; else thousand=temp/1000; hundred=(temp%1000)/100; tens=(temp%100)/10; ones=temp%10; level:; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x79, 0x40}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} | 08 Perform any digit Multiplication. If the total exceeds 9999 then display “EEEE”. Ex: 25 x 40 = 1000, 1111 x 8 = 8888 :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile short number=0, temp, num1, num2; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if(key=='+' ¦¦ key=='-' ¦¦ key=='*' ¦¦ key=='/'){ num1=number; number=0;} else if(key=='='){ num2=number; number=num1*num2;} else if(number==0) number=key; else number = (number*10) + key; if(key=='c') number=num1=num2=0; if(key=='=' && number>9999){thousand=hundred=tens=ones=10; goto level;} temp=number; temp%=10000; thousand=temp/1000; hundred=(temp%1000)/100; tens=(temp%100)/10; ones=temp%10; level:; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x79}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} |
| --- | --- |

| 09 Perform any digit Division. Display only Quotient. Ex: 1561 / 6 = 260, 765 / 9 = 85 :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile long number=0, temp, num1, num2; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if(key=='+' ¦¦ key=='-' ¦¦ key=='*' ¦¦ key=='/'){ num1=number; number=0;} else if(key=='='){ num2=number; number=num1/num2;} else if(number==0) number=key; else number = (number*10) + key; if(key=='c') number=num1=num2=0; if(key=='=' && number>9999){thousand=hundred=tens=ones=10; goto level;} temp=number; temp%=10000; thousand=temp/1000; hundred=(temp%1000)/100; tens=(temp%100)/10; ones=temp%10; level:; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x79}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} | 10 Perform any digit Division. Display only Remainder. Ex: 1561 / 6 = 1, 765 / 9 = 0 :- volatile char checkKey(); void calculation(volatile char key); void display(); void init(); volatile char *dirf, *outf, *dirk, *outk, *dira, *outa, *dirc, *inc; volatile char ones, tens, hundred, thousand; volatile char flag=0; volatile long number=0, temp, num1, num2; void setup() { volatile char key; volatile long i; init(); while(1){ key=checkKey(); if(key){ for(i=0; i<10000; i++) display(); calculation(key); } display(); } } void calculation(volatile char key){ if(key=='0') key=0; if(key=='+' ¦¦ key=='-' ¦¦ key=='*' ¦¦ key=='/'){ num1=number; number=0;} else if(key=='='){ num2=number; number=num1%num2;} else if(number==0) number=key; else number = (number*10) + key; if(key=='c') number=num1=num2=0; if(key=='=' && number>9999){thousand=hundred=tens=ones=10; goto level;} temp=number; temp%=10000; thousand=temp/1000; hundred=(temp%1000)/100; tens=(temp%100)/10; ones=temp%10; level:; } void display(){ if(flag==1) goto level; volatile char arr[]={0x3f,0x06,0x5b,0x4f,0x66,0x6d, 0x7d,0x07,0x7f, 0x6f, 0x79}; volatile char d4=~1, d3=~2, d2=~4, d1=~8, clr=~16; flag=1; level: *outk=d4; *outf=arr[thousand]; *outk=d4; *outf=0; *outk=d3; *outf=arr[hundred]; *outk=d3; *outf=0; *outk=d2; *outf=arr[tens]; *outk=d2; *outf=0; *outk=d1; *outf=arr[ones]; *outk=d1; *outf=0; } volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ *outa= 1 << row; if(*inc){ col=*inc; break;} } if(row==0){ if(col==1) key=1; else if(col==2) key=2; else if(col==4) key=3; else if(col==8) key='+'; } else if(row==1){ if(col==1) key=4; else if(col==2) key=5; else if(col==4) key=6; else if(col==8) key='-'; } else if(row==2){ if(col==1) key=7; else if(col==2) key=8; else if(col==4) key=9; else if(col==8) key='*'; } else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='/'; } else key=0; return key; } void init(){ dira=0x21; *dira=0xff; outa=0x22; dirc=0x27; *dirc=0; inc=0x26; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; } void loop() {} |
| --- | --- |

ESP – Dot-matrix – Problems Set - 1

| 01 Display Right most Top LED :- void setup() { volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0xff; outf=0x31; outk=0x108; *outf=1; *outk=~1; } void loop() {} |  |
| --- | --- |
| 02 Display Left most Top LED :- void setup() { volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0xff; outf=0x31; outk=0x108; *outf=128; *outk=~1; } void loop() {} | 03 Display Right most bottom LED :- void setup() { volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0xff; outf=0x31; outk=0x108; *outf=1; *outk=~128; } void loop() {} |
| 04 Display Left most bottom LED :- void setup() { volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0xff; outf=0x31; outk=0x108; *outf=128; *outk=~128; } void loop() {} | 05 Display Topmost Row all LEDs :- void setup() { volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0xff; outf=0x31; outk=0x108; *outf=0xff; *outk=~1; } void loop() {} |

| 06 Display Bottom most Row all LEDs :- void setup() { volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0xff; outf=0x31; outk=0x108; *outf=0xff; *outk=~128; } void loop() {} |  |
| --- | --- |
| 07 Display Left most Column all LEDs :- void setup() { volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0xff; outf=0x31; outk=0x108; *outf=128; *outk=0; } void loop() {} | 08 Display Right most Column all LEDs :- void setup() { volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0xff; outf=0x31; outk=0x108; *outf=1; *outk=0; } void loop() {} |
| 09 Display diagonal LEDs connecting Left top LED to Right bottom LED :- void setup() { volatile char *dirf, *dirk, *outf, *outk, row, col; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; while(1){ for(row=0; row<9; row++) for(col=0; col<9; col++) if(row==col){ *outf= 1 << row; *outk= ~( 128 >> col); *outf=0; *outk=0xff; } } } void loop() {} | 10 Display diagonal LEDs connecting Right top LED to Left bottom LED :- void setup() { volatile char *dirf, *dirk, *outf, *outk, row, col; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; while(1){ for(row=0; row<9; row++) for(col=0; col<9; col++) if(row==col){ *outf= 1 << row; *outk= ~( 1 << col); *outf=0; *outk=0xff; } } } void loop() {} |

ESP – Dot-matrix – Problems Set – 2

| 01 Display Character A :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00000000, 0b00011000, 0b00100100, 0b01111110, 0b10000001, 0b00000000, 0b00000000, 0b00000000}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} |  |
| --- | --- |
| 02 Display Character B :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b11111100, 0b10000010, 0b10000010, 0b11111100, 0b10000010, 0b10000010, 0b11111100, 0b00000000}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} | 03 Display Number 1 :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00011000, 0b00111000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00111100}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} |

| 04 Display Number 2 :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00111110, 0b01111110, 0b01100110, 0b00001110, 0b00011100, 0b00111000, 0b01111111, 0b01111111}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} | 05 Display Character a :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00011000, 0b00100100, 0b00000100, 0b00011100, 0b00100100, 0b00100100, 0b00011010, 0b00000000}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} |
| --- | --- |
| 06 Display Character q :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00111111, 0b01111111, 0b01100011, 0b01111111, 0b00111111, 0b00000011, 0b00000011, 0b00000011}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} | 07 Display Symbol $ :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00010000, 0b00111000, 0b01010100, 0b01010000, 0b00111000, 0b00010100, 0b01010100, 0b00111000}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} |

| 08 Display Symbol # :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00000000, 0b00010100, 0b01111110, 0b00101000, 0b11111100, 0b01010000, 0b00000000, 0b00000000}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} |  |
| --- | --- |
| 09 Display Symbol α :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00000000, 0b00000000, 0b00011010, 0b00100100, 0b00100100, 0b00011010, 0b00000000, 0b00000000}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} | 10 Display Symbol Ω :- void setup() { volatile char i, *dirf, *dirk, *outf, *outk; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char num[] = {0b00000000, 0b00000000, 0b00111000, 0b01000100, 0b01000100, 0b00101000, 0b11101110, 0b00000000}; while(1){ for(i=0; i<8; i++){ *outk= ~(1 << i); *outf = num[i]; *outk=0xff; *outf=0; } } } void loop() {} |

| 01 Display Characters A to E one by one in one second Interval :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char abcde[][8] = { {0b00000000, 0b00011000, 0b00100100, 0b01111110, 0b10000001, 0b00000000, 0b00000000, 0b00000000}, // A {0b01111000, 0b01000100, 0b01000100, 0b01111000, 0b01000100, 0b01000100, 0b01111000, 0b00000000}, // B {0b00000000, 0b00111000, 0b01000100, 0b01000000, 0b01000000, 0b01000100, 0b00111000, 0b00000000}, // C {0b00000000, 0b01111000, 0b01000100, 0b01000100, 0b01000100, 0b01000100, 0b01111000, 0b00000000}, // D {0b00000000, 0b01111100, 0b01000000, 0b01111100, 0b01000000, 0b01111100, 0b00000000, 0b00000000}}; // E while(1){ for(a=0; a<20000; a++){ for(i=0; i<8; i++) { *outk= ~(1 << i); *outf = ecen[count][i]; *outk=0xff; *outf=0; } } for(a=0; a<45000; a++); count++; if(count==5) count=0; } } void loop() {} | 02 Display Characters a to e one by one in one second Interval :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char ecen[][8] = { {0b00011000, 0b00100100, 0b00000100, 0b00011100, 0b00100100, 0b00100100, 0b00011010, 0b00000000}, // a {0b00100000, 0b00100000, 0b00100000, 0b00111000, 0b00100100, 0b00100100, 0b00011000, 0b00000000}, // b {0b00000000, 0b00011000, 0b00100100, 0b00100000, 0b00100000, 0b00100100, 0b00011000, 0b00000000}, // c {0b00000100, 0b00000100, 0b00000100, 0b00011100, 0b00100100, 0b00100100, 0b00011000, 0b00000000}, //d {0b00000000, 0b00011000, 0b00100100, 0b00111100, 0b00100000, 0b00011000, 0b00000000, 0b00000000}}; //e while(1){ for(a=0; a<20000; a++){ for(i=0; i<8; i++) { *outk= ~(1 << i); *outf = ecen[count][i]; *outk=0xff; *outf=0; } } for(a=0; a<45000; a++); count++; if(count==5) count=0; } } void loop() {} |
| --- | --- |

ESP – Dot-matrix – Problems Set – 3

| 03 Display Numbers 0 to 4 one by one in one second Interval :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char ecen[][8] = { {0b00010000, 0b00110000, 0b00010000, 0b00010000, 0b00010000, 0b00010000, 0b00010000, 0b00111000}, // 1 {0b00111100, 0b01000010, 0b01000100, 0b00001000, 0b00010000, 0b00100000, 0b01111110, 0b00000000}, // 2 {0b00111110, 0b00000100, 0b00001000, 0b00011100, 0b00000010, 0b01000010, 0b00111100, 0b00000000}, // 3 {0b00000000, 0b00010100, 0b00100100, 0b01000100, 0b01111110, 0b00000100, 0b00000100, 0b00000000}, // 4 {0b00000000, 0b00111100, 0b00100000, 0b00111000, 0b00000100, 0b00100100, 0b00011000, 0b00000000}}; while(1){ for(a=0; a<20000; a++){ for(i=0; i<8; i++) { *outk= ~(1 << i); *outf = ecen[count][i]; *outk=0xff; *outf=0; } } for(a=0; a<45000; a++); count++; if(count==5) count=0; } } void loop() {} | 04 Display Welcome string characters one by one in one second Interval :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a; char i, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char ecen[][8] = { {0b11000011, 0b11000011, 0b11000011, 0b11000011, 0b11011011, 0b11111111, 0b11100111, 0b11000011}, // W {0b00000000, 0b00000000, 0b00000000, 0b00111100, 0b01100110, 0b01111110, 0b01100000, 0b00111100}, //e {0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00001100}, // l {0b00000000, 0b00000000, 0b00000000, 0b00111100, 0b01101100, 0b01100000, 0b01100000, 0b00111100}, //c {0b00000000, 0b00000000, 0b00000000, 0b00111100, 0b01100110, 0b01100110, 0b01100110, 0b00111100}, //o {0b00000000, 0b00000000, 0b00000000, 0b01110110, 0b11011011, 0b11011011, 0b11011011, 0b11011011}, // m {0b00000000, 0b00000000, 0b00000000, 0b00111100, 0b01100110, 0b01111110, 0b01100000, 0b00111100}}; //e while(1){ for(a=0; a<20000; a++){ for(i=0; i<8; i++) { *outk= ~(1 << i); *outf = ecen[count][i]; *outk=0xff; *outf=0; } } for(a=0; a<45000; a++); count++; if(count==7) count=0; } } void loop() {} |
| --- | --- |

| 05 Display revolving text “Welcome” Right to Left :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a, b; char i, j, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char arr[] = { 0, 0, 0, 0, 0, 0, 0, 0, 0b11111111, 0b11111111, 0b01100000, 0b00110000, 0b00110000, 0b01100000, 0b11111111, 0b11111111, // W 0b00000000, 0b01110000, 0b11111000, 0b10101000, 0b10101000, 0b10111000, 0b00110000, 0b00000000, // e //0b00000000, 0b00000000, 0b00000000, 0b01111111, 0b11111111, 0b10000000, //0b00000000, 0b00000000, // l 0b00000000, 0b01110000, 0b11111000, 0b10001000, 0b10011000, 0b10011000, //0b00000000, 0b00000000, // c 0b00000000, 0b01110000, 0b11111000, 0b10001000, 0b10001000, 0b11111000, 0b01110000, 0b00000000, // o 0b11110000, 0b11111000, 0b00001000, 0b11111000, 0b11110000, 0b00001000, 0b11111000, 0b11110000, // m 0b00000000, 0b01110000, 0b11111000, 0b10101000, 0b10101000, 0b10111000, 0b00110000, 0b00000000, // e 0, 0, 0, 0, 0, 0, 0, 0, }; while(1){ for(i=0; i<57; i++){ for(a=0; a<8000; a++){ for(j=0; j<8; j++){ *outf= 128 >> j; *outk= ~(arr[i+j]); *outk=0xff; *outf=0; } } for(b=0; b<32000; b++); } } } void loop() {} | 06 Display revolving text “Welcome” Left to Right :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a, b; char i, j, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char arr[] = { 0, 0, 0, 0, 0, 0, 0, 0, 0b00000000, 0b01110000, 0b11111000, 0b10101000, 0b10101000, 0b10111000, 0b00110000, 0b00000000, // e 0b11110000, 0b11111000, 0b00001000, 0b11111000, 0b11110000, 0b00001000, 0b11111000, 0b11110000, // m 0b00000000, 0b01110000, 0b11111000, 0b10001000, 0b10001000, 0b11111000, 0b01110000,// 0b00000000, // o 0b00000000, 0b01110000, 0b11111000, 0b10001000, 0b10011000, 0b10011000, 0b00000000, //0b00000000, // c //0b00000000, 0b00000000, 0b00000000, 0b01111111, 0b11111111, 0b10000000, //0b00000000, 0b00000000, // l 0b00000000, 0b01110000, 0b11111000, 0b10101000, 0b10101000, 0b10111000, 0b00110000, 0b00000000, // e 0b11111111, 0b11111111, 0b01100000, 0b00110000, 0b00110000, 0b01100000, 0b11111111, 0b11111111, // W 0, 0, 0, 0, 0, 0, 0, 0, }; while(1){ for(i=56; i>=0; i--){ for(a=0; a<8000; a++){ for(j=0; j<8; j++){ *outf= 128 >> j; *outk= ~(arr[i+j]); *outk=0xff; *outf=0; } } for(b=0; b<32000; b++); } } } void loop() {} |
| --- | --- |

| 07 Display revolving text “Welcome” Top to Bottom :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a, b; char i, j, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char arr[] = { 0, 0, 0, 0, 0, 0, 0, 0, 0b11000011, 0b11100111, 0b11111111, 0b11011011, 0b11000011, 0b11000011, 0b11000011, 0b11000011, 0, // W 0b00111100, 0b01100000, 0b01111110, 0b01100110, 0b00111100, 0b00000000, // 0b00000000, 0b00000000, //e 0b00001100, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0, // l 0b00111100, 0b01100000, 0b01100000, 0b01101100, 0b00111100, 0b00000000, // 0b00000000, 0b00000000, //c 0b00111100, 0b01100110, 0b01100110, 0b01100110, 0b00111100, 0b00000000, // 0b00000000, 0b00000000, //o 0b11011011, 0b11011011, 0b11011011, 0b11011011, 0b01110110, 0b00000000, // 0b00000000, 0b00000000, // m 0b00111100, 0b01100000, 0b01111110, 0b01100110, 0b00111100, 0b00000000, //0b00000000, 0b00000000, //e 0, 0, 0, 0, 0, 0, 0, 0, }; while(1){ for(i=0; i<58; i++){ for(a=0; a<8000; a++){ for(j=0; j<8; j++){ *outk= ~(128 >> j); *outf= arr[i+j]; *outk=0xff; *outf=0; } } for(b=0; b<32000; b++); } } } void loop() {} | 08 Display revolving text “Welcome” Bottom to Top :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a, b; char i, j, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char arr[] = { 0, 0, 0, 0, 0, 0, 0, 0, 0b11000011, 0b11000011, 0b11000011, 0b11000011, 0b11011011, 0b11111111, 0b11100111, 0b11000011, // W 0b00000000, // 0b00000000, 0b00000000, 0b00111100, 0b01100110, 0b01111110, 0b01100000, 0b00111100, 0, //e 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00001100, // l 0b00000000, // 0b00000000, 0b00000000, 0b00111100, 0b01101100, 0b01100000, 0b01100000, 0b00111100, //c 0b00000000, // 0b00000000, 0b00000000, 0b00111100, 0b01100110, 0b01100110, 0b01100110, 0b00111100, //o 0b00000000, // 0b00000000, 0b00000000, 0b01110110, 0b11011011, 0b11011011, 0b11011011, 0b11011011, // m 0b00000000, // 0b00000000, 0b00000000, 0b00111100, 0b01100110, 0b01111110, 0b01100000, 0b00111100, //e 0, 0, 0, 0, 0, 0, 0, 0 }; while(1){ for(i=0; i<57; i++){ for(a=0; a<8000; a++){ for(j=0; j<8; j++){ *outk= ~(1 << j); *outf= arr[i+j]; *outk=0xff; *outf=0; } } for(b=0; b<32000; b++); } } } void loop() {} |
| --- | --- |

| 09 Display “Welcome” by odd position entering from Left and Even Position entering from Right :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a, b; char i, j, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char arr[][24] = { {0, 0, 0, 0, 0, 0, 0, 0, 0b11111111, 0b11111111, 0b01100000, 0b00110000, 0b00110000, 0b01100000, 0b11111111, 0b11111111, 0, 0, 0, 0, 0, 0, 0, 0}, // W {0, 0, 0, 0, 0, 0, 0, 0, 0b00000000, 0b01110000, 0b11111000, 0b10101000, 0b10101000, 0b10111000, 0b00110000, 0b00000000, 0, 0, 0, 0, 0, 0, 0, 0}, // e {0, 0, 0, 0, 0, 0, 0, 0, 0b00000000, 0b00000000, 0b00000000, 0b01111111, 0b11111111, 0b10000000, 0b00000000, 0b00000000, 0, 0, 0, 0, 0, 0, 0, 0}, // l {0, 0, 0, 0, 0, 0, 0, 0, 0b00000000, 0b01110000, 0b11111000, 0b10001000, 0b10011000, 0b10011000, 0b00000000, 0b00000000, 0, 0, 0, 0, 0, 0, 0, 0}, // c {0, 0, 0, 0, 0, 0, 0, 0, 0b00000000, 0b01110000, 0b11111000, 0b10001000, 0b10001000, 0b11111000, 0b01110000, 0b00000000, 0, 0, 0, 0, 0, 0, 0, 0}, // o {0, 0, 0, 0, 0, 0, 0, 0, 0b11110000, 0b11111000, 0b00001000, 0b11111000, 0b11110000, 0b00001000, 0b11111000, 0b11110000, 0, 0, 0, 0, 0, 0, 0, 0}, // m {0, 0, 0, 0, 0, 0, 0, 0, 0b00000000, 0b01110000, 0b11111000, 0b10101000, 0b10101000, 0b10111000, 0b00110000, 0b00000000, 0, 0, 0, 0, 0, 0, 0, 0}}; // e while(1){ loop: for(i=16; i>=0; i--){ for(a=0; a<8000; a++){ for(j=0; j<8; j++){ *outf= 128 >> j; *outk= ~(arr[count][i+j]); *outk=0xff; *outf=0; } } for(b=0; b<32000; b++); } count++; if(count>6){count=0; goto loop;} for(i=0; i<17; i++){ for(a=0; a<8000; a++){ for(j=0; j<8; j++){ *outf= 128 >> j; *outk= ~(arr[count][i+j]); *outk=0xff; *outf=0; } } for(b=0; b<32000; b++); } count++; } } void loop() {} | 10 Display “Welcome” by odd position entering from Top and Even Position entering from Bottom :- void setup(){ volatile char *dirf, *dirk, *outf, *outk; volatile long a, b; char i, j, count=0; dirf=0x30; *dirf=0xff; outf=0x31; dirk=0x107; *dirk=0xff; outk=0x108; volatile char arr[][24] = { {0, 0, 0, 0, 0, 0, 0, 0, 0b11000011, 0b11100111, 0b11111111, 0b11011011, 0b11000011, 0b11000011, 0b11000011, 0b11000011, // W 0, 0, 0, 0, 0, 0, 0, 0}, {0, 0, 0, 0, 0, 0, 0, 0, 0b00111100, 0b01100000, 0b01111110, 0b01100110, 0b00111100, 0b00000000, 0b00000000, 0b00000000, //e 0, 0, 0, 0, 0, 0, 0, 0}, {0, 0, 0, 0, 0, 0, 0, 0, 0b00001100, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, 0b00011000, // l 0, 0, 0, 0, 0, 0, 0, 0}, {0, 0, 0, 0, 0, 0, 0, 0, 0b00111100, 0b01100000, 0b01100000, 0b01101100, 0b00111100, 0b00000000, 0b00000000, 0b00000000, //c 0, 0, 0, 0, 0, 0, 0, 0}, {0, 0, 0, 0, 0, 0, 0, 0, 0b00111100, 0b01100110, 0b01100110, 0b01100110, 0b00111100, 0b00000000, 0b00000000, 0b00000000, //o 0, 0, 0, 0, 0, 0, 0, 0}, {0, 0, 0, 0, 0, 0, 0, 0, 0b11011011, 0b11011011, 0b11011011, 0b11011011, 0b01110110, 0b00000000, 0b00000000, 0b00000000, // m 0, 0, 0, 0, 0, 0, 0, 0}, {0, 0, 0, 0, 0, 0, 0, 0, 0b00111100, 0b01100000, 0b01111110, 0b01100110, 0b00111100, 0b00000000, 0b00000000, 0b00000000, //e 0, 0, 0, 0, 0, 0, 0, 0}}; while(1){ loop: for(i=0; i<17; i++){ for(a=0; a<8000; a++){ for(j=0; j<8; j++){ *outk= ~(128 >> j); *outf= arr[count][i+j]; *outk=0xff; *outf=0; } } for(b=0; b<32000; b++); } count++; if(count>6){count=0; goto loop;} for(i=16; i>=0; i--){ for(a=0; a<8000; a++){ for(j=0; j<8; j++){ *outk= ~(128 >> j); *outf= arr[count][i+j]; *outk=0xff; *outf=0; } } for(b=0; b<32000; b++); } count++; } } void loop() {} |
| --- | --- |