- ESP – LCD Display Programming (Lecture) :-

| volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ init_port(); init_lcd(); lcd_data("Welcome to ECEN"); } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display lcd_cmd(0x06); // cursor auto increment } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |  |
| --- | --- |

ESP – LCD Display (16 x 2) Set - 1

| 01 Display “Welcome" on line1. Left Justified :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ init_port(); init_lcd(); lcd_data("Welcome"); } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display lcd_cmd(0x06); // cursor auto increment } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 02 Display “Welcome” on line1. Right Justified:- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ init_port(); init_lcd(); lcd_data("Welcome"); } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display lcd_cmd(0x06); // cursor auto increment lcd_cmd(0x89); // set cursor to row 0 col 9 } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

| 03 Display “Welcome" on line1. Middle :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ init_port(); init_lcd(); lcd_data("Welcome"); } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display lcd_cmd(0x06); // cursor auto increment lcd_cmd(0x84); // set cursor to row 0 col 4 } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 04 Display “Welcome" on line2. Left Justified :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ init_port(); init_lcd(); lcd_data("Welcome"); } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display lcd_cmd(0x06); // cursor auto increment lcd_cmd(0xc0); // set cursor to row 1 col 0 } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

| 05 Display “Welcome" on line2. Right Justified:- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ init_port(); init_lcd(); lcd_data("Welcome"); } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display lcd_cmd(0x06); // cursor auto increment lcd_cmd(0xc9); // set cursor to row 1 col 9 } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 06 Display “Welcome" on line2. Middle :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ init_port(); init_lcd(); lcd_data("Welcome"); } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display lcd_cmd(0x06); // cursor auto increment lcd_cmd(0xc4); // set cursor to row 1 col 4 } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

| 07 Display “Welcome" on line1. Middle & Display “To ECEN Academy” on line2 Middle :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ init_port(); init_lcd(); lcd_cmd(0x84); // set cursor to row 0 col 4 lcd_data("Welcome"); lcd_cmd(0xc0); // set cursor to row 1 col 0 lcd_data("to ECEN Academy"); } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 08 Display “Welcome” on line1. Enter from Right. Left Justified :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a; volatile long i; init_port(); init_lcd(); for(a=0x8f; a>=0x80; a--){ lcd_cmd(0x01); lcd_cmd(a); lcd_data("Welcome"); for(i=0; i<100000; i++); } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

| 09 Display “Welcome” on line2. Enter from Left. Right Justified :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a; volatile long i; init_port(); init_lcd(); for(a=0xb9; a<=0xc9; a++){ lcd_cmd(0x01); lcd_cmd(a); lcd_data("Welcome"); for(i=0; i<100000; i++); } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); // rs=1; en=1; *outk=2; delay1(1); // rs=1; en=0; ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); // rs=0; en=1; *outk=0; delay1(1); // rs=0; en=0; } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 10 Display “Welcome” on Line 1. Enter from Left to Middle. Display “To ECEN Academy” on Line2 Enter from Right to Middle :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a; volatile long i; init_port(); init_lcd(); for(a=0x80; a<0x8c; a++){ lcd_cmd(0x01); lcd_cmd(0x04); // for left shift the cursor; lcd_cmd(a); lcd_data("emocleW"); for(i=0; i<100000; i++); } for(a=0xd0; a>=0xc1; a--){ lcd_cmd(0x01); lcd_cmd(0x85); lcd_data("Welcome"); lcd_cmd(a); lcd_data("To ECEN Academy"); for(i=0; i<100000; i++); } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); lcd_cmd(0x0f); lcd_cmd(0x01); } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

ESP – LCD Display (16 x 2) Set - 2

| 01 Display “Welcome to" on line1. Rotate left to right continuously :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile char a; volatile long i; init_port(); init_lcd(); while(1){ lcd_cmd(0x0a); // Display off cursor on lcd_data("Welcome To"); lcd_cmd(0x0c); // Display on cursor off // shifting entire display left 10 position for(a=0; a<10; a++) lcd_cmd(0x18); for(a=0; a<25; a++){ lcd_cmd(0x1c); for(i=0; i<100000; i++); } lcd_cmd(0x01); } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 02 Display “Welcome to" on line1. Rotate right to left continuously :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile char a; volatile long i; init_port(); init_lcd(); while(1){ lcd_cmd(0x0a); // Display off cursor on lcd_cmd(0x91); lcd_data("Welcome To"); lcd_cmd(0x0c); // Display on cursor off for(a=0; a<25; a++){ lcd_cmd(0x18); for(i=0; i<100000; i++); } lcd_cmd(0x01); } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

| 03 Display “ECEN Academy" on line2. Rotate left to right continuously :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile char a; volatile long i; init_port(); init_lcd(); while(1){ lcd_cmd(0x0a); // Display off cursor on lcd_cmd(0xc0); lcd_data("ECEN Academy"); // shifting entire display left 10 position for(a=0; a<12; a++) lcd_cmd(0x18); lcd_cmd(0x0c); // Display on cursor off for(a=0; a<29; a++){ lcd_cmd(0x1c); for(i=0; i<100000; i++); } lcd_cmd(1); } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 04 Display “ECEN Academy" on line2. Rotate right to left continuously :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile char a; volatile long i; init_port(); init_lcd(); while(1){ lcd_cmd(0x0a); // Display off cursor on lcd_cmd(0xd1); lcd_data("ECEN Academy"); lcd_cmd(0x0c); // Display on cursor off for(a=0; a<29; a++){ lcd_cmd(0x18); for(i=0; i<100000; i++); } lcd_cmd(0x01); } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); // rs=1; en=0; *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); // 8 bit bus & 2 rows function set lcd_cmd(0x0f); // display on, cursor blink lcd_cmd(0x01); // clear display } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); // rs=0; en=0; *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

| 05 Display “Welcome to" on line1 in middle and “ECEN Academy” on line2. Rotate line2 right to left continuously :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a; volatile long i; init_port(); init_lcd(); while(1){ for(a=0xd0; a>=0xb5; a--){ lcd_cmd(0x01); lcd_cmd(0x83); lcd_data("Welcome To"); lcd_cmd(a); lcd_data("ECEN Academy"); for(i=0; i<100000; i++); } } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); lcd_cmd(0x0c); lcd_cmd(0x01); } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 06 Display “Welcome to" on line1. Rotate line1 right to left Continuously and “ECEN Academy” on line2 in Middle :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a; volatile long i; init_port(); init_lcd(); while(1){ for(a=0x9a; a>=0x80; a--){ lcd_cmd(0x01); lcd_cmd(0x04); // for left shift the cursor; lcd_cmd(a); lcd_data("oT emocleW"); lcd_cmd(0xcd); lcd_data("ymedacA NECE"); for(i=0; i<100000; i++); } } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); lcd_cmd(0x0c); lcd_cmd(0x01); } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

| 07 Display “Welcome to" on line1 in middle. “ECEN Academy” on line2 and rotate line2 left to right continuously :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a; volatile long i; init_port(); init_lcd(); while(1){ for(a=0xc0; a<=0xdb; a++){ lcd_cmd(0x01); lcd_cmd(0x83); lcd_data("Welcome To"); lcd_cmd(0x04); // for left shift the cursor; lcd_cmd(a); lcd_data("ymedacA NECE"); for(i=0; i<100000; i++); } } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); lcd_cmd(0x0c); lcd_cmd(0x01); } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 08 Display “Welcome to" on line1. Rotate line1 left to right Continuously and “ECEN Academy” on line2 in Middle :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a; volatile long i; init_port(); init_lcd(); while(1){ for(a=0x80; a<=0x9b; a++){ lcd_cmd(0x01); lcd_cmd(0x04); // for left shift the cursor; lcd_cmd(a); lcd_data("oT emocleW"); lcd_cmd(0x06); // for right shift the cursor lcd_cmd(0xc2); lcd_data("ECEN Academy"); for(i=0; i<100000; i++); } } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); lcd_cmd(0x0c); lcd_cmd(0x01); } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |

| 09 Display “Welcome to" on line1. Rotate right to left continuously. “ECEN Academy" on line2. Rotate left to right continuously :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a, b; volatile long i; init_port(); init_lcd(); while(1){ for(a=0x9a, b=0xc0; a>=0x80; a--, b++){ lcd_cmd(0x01); lcd_cmd(0x04); // for left shift the cursor; lcd_cmd(a); lcd_data("oT emocleW"); lcd_cmd(0x04); // for left shift the cursor; lcd_cmd(b); lcd_data("ymedacA NECE"); for(i=0; i<100000; i++); } } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); lcd_cmd(0x0c); lcd_cmd(0x01); } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} | 10 Display “Welcome to" on line1. Rotate left to right continuously. “ECEN Academy" on line2. Rotate right to left continuously :- volatile char *dirf, *outf, *dirk, *outk; void init_port(); void delay1(int); void init_lcd(); void lcd_cmd(volatile char cmd); void lcd_data(volatile char *ptr); void setup(){ volatile short a, b; volatile long i; init_port(); init_lcd(); while(1){ for(a=0x80, b=0xda; a<=0x9b; a++, b--){ lcd_cmd(0x01); lcd_cmd(0x04); // for left shift the cursor; lcd_cmd(a); lcd_data("oT emocleW"); lcd_cmd(b); lcd_data("ymedacA NECE"); for(i=0; i<100000; i++); } } } void lcd_data(volatile char *ptr){ while(*ptr){ *outf=*ptr; *outk=2; delay1(1); *outk=3; delay1(1); *outk=2; delay1(1); ptr++; } } void init_lcd(){ lcd_cmd(0x38); lcd_cmd(0x0c); lcd_cmd(0x01); } void lcd_cmd(volatile char cmd){ *outf=cmd; *outk=0; delay1(1); *outk=1; delay1(1); *outk=0; delay1(1); } void init_port(){ dirf=0x30; *dirf=0xff; dirk=0x107; *dirk=0x03; outf=0x31; outk=0x108; } void delay1(volatile int time){ volatile long i; while(time--) for(i=0; i<100; i++); } void loop() {} |
| --- | --- |