CONTROLLER PROGRAMMING

| 01. Timer1 Programming using Interrupt ( From Lecture):- void init_port(); void init_timer(); void setup() { volatile long i; int j=0; init_port(); init_timer(); Serial.begin(9600); while(1) { for(i=0; i<500000; i++); Serial.println(j,DEC); j++; } } void init_timer(){ volatile char *Timer1_TCCR1A = 0x80; // Timer/Counter 1 Control Register A volatile char *Timer1_TCCR1B = 0x81; // Timer/Counter 1 Control Register B volatile short *Timer1_TCNT1 = 0x84; // Timer/Counter 1 Register volatile short *Timer1_OCR1A = 0x88; // Output Compare Register 1 A volatile char *Timer1_TIMSK1 = 0x6F; // Timer/Counter 1 Interrupt Mask register *Timer1_TCCR1A = 0; //*Timer1_TCCR1B = 0; *Timer1_TCCR1B = 0x0c; // TCCR1A & TCCR1B are assigned such that the timer 1 acts as CTC mode and clk/256 prescaler mode *Timer1_TCNT1 = 0; // count 0 to OCR1A value *Timer1_OCR1A = 60000; // our required value *Timer1_TIMSK1 = 0x02; // set the interrupt mask register OCIE1A bit } | void init_port(){ volatile char *portf_dir = (volatile char *) 0x30; *portf_dir = 0x01; } ISR(TIMER1_COMPA_vect){ volatile char *portf_data = (volatile char *) 0x31; *portf_data ^= 0x01; } void loop() {} |
| --- | --- |

Timer0 & Timer2 (8-bit Timer/Counter) Programming

| 02. Timer1 & Timer2 Programming using Interrupt ( From Lecture):- void init_port(); void init_timer1(); void init_timer3(); volatile char *portf_data; volatile char *portk_data; void setup() { volatile long i; int j=0; init_port(); init_timer1(); init_timer3(); Serial.begin(9600); while(1) { for(i=0; i<500000; i++); Serial.println(j,DEC); j++; } } void init_timer3(){ // By using Normal mode volatile char *Timer3_TCCR3A = 0x90; volatile char *Timer3_TCCR3B = 0x91; volatile short *Timer3_TCNT3 = 0x94; volatile char *Timer3_TIFR3 = 0x71; *Timer3_TCCR3A = 0; *Timer3_TCCR3B = 0x04; *Timer3_TCNT3 = 0; *Timer3_TIFR3 = 1; } void init_timer1(){ // By using CTC mode volatile char *Timer1_TCCR1A = 0x80; volatile char *Timer1_TCCR1B = 0x81; volatile short *Timer1_TCNT1 = 0x84; volatile short *Timer1_OCR1A = 0x88; volatile char *Timer1_TIMSK1 = 0x6F; | *Timer1_TCCR1A = 0; //*Timer1_TCCR1B = 0; *Timer1_TCCR1B = 0x0c; // TCCR1A & TCCR1B are assigned such that the timer 1 acts as CTC mode and clk/256 prescaler mode *Timer1_TCNT1 = 0; // count 0 to OCR1A value *Timer1_OCR1A = 8000; // our required value *Timer1_TIMSK1 = 0x02; // set the interrupt mask register OCIE1A bit } void init_port(){ volatile char *portf_dir = 0x30, *portk_dir = 0x107; *portf_dir = 0x01; *portk_dir = 1; portf_data = 0x31; portk_data = 0x108; } ISR(TIMER1_COMPA_vect){ *portf_data ^= 0x01; } ISR(TIMER3_OVF_vect){ *portk_data ^= 0x01; } void loop() {} |
| --- | --- |

| 03. Timer0 programming in Normal Mode:- ( Another program of Normal Mode using Interrupt are at the last of the 8-bit Timer/Counter ) uint8_t i=0; void setup() { Serial.begin(9600); DDRB = 1; TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; while(1){ if(TCNT0>=255){ i++; if(i%20==0){ PORTB ^= 1; Serial.println(i); } TCNT0=0; } } } void loop() {} | From 03, all program written by myself |
| --- | --- |
| 04. Timer0 programming in CTC Mode, OC0A pin toggling on compare match:- uint8_t count_compare; void setup() { Serial.begin(9600); DDRD ¦= 64; // Set pin6 as Output TCCR0A = 64+2; // 64 for toggling OC0A pin on compare match & 2 for CTC mode TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0A = 255; while(1){ if(TCNT0==250){ count_compare++; Serial.println(count_compare); } } } void loop() {} // Output > Toggled the OC0A pin (i.e., Arduino pin 6) | For OC0A Pin Operation |

| 05. Timer0 programming in CTC Mode, OC0B pin toggling on compare match:- uint8_t count_compare; void setup() { Serial.begin(9600); DDRD ¦= 32; // Set pin5 as Output TCCR0A = 2 + 16; // 16 for toggling OC0B pin on compare match & 2 for CTC mode TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0A = 255; while(1){ if(TCNT0==250){ count_compare++; Serial.println(count_compare); } } } void loop() {} // Output > Toggled the OC0B pin (i.e., Arduino pin 5) | For OC0B Pin Operation |
| --- | --- |
| 06. Timer0 programming in Fast PWM (Inverting) Mode for OC0A pin:- void setup() { DDRD ¦= 64; // Set pin6 as Output TCCR0A = 128+64+2+1; // 128+64 for inverting mode PWM & 2+1 for Fast PWM generation // Inverting mode means set OC0A on compare match and clear OC0A at bottom TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0A = 255; // According to the value of OCR0A, output will change while(1){} } void loop() {} // Output > PWM output at OC0A pin (i.e., Arduino pin 6) | 07. Timer0 programming in Fast PWM (Non - Inverting) Mode for OC0A pin:- void setup() { DDRD ¦= 64; // Set pin6 as Output TCCR0A = 128+2+1; // 128 for non-inverting mode PWM & 2+1 for Fast PWM generation // Non-inverting mode means clear OC0A on compare match and set OC0A at bottom TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0A = 25; // According to the value of OCR0A, output will change while(1){} } void loop() {} // Output > PWM output at OC0A pin (i.e., Arduino pin 6) |

| 08. Timer0 programming in Fast PWM (Inverting) Mode for OC0B pin:- void setup() { DDRD ¦= 32; // Set pin5 as Output TCCR0A = 32+16+2+1; // 32+16 for inverting mode PWM & 2+1 for Fast PWM generation // Inverting mode means set OC0B on compare match and clear OC0B at bottom TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0B = 255; // According to the value of OCR0B, output will change while(1){} } void loop() {} // Output > PWM output at OC0B pin (i.e., Arduino pin 5) | 09. Timer0 programming in Fast PWM (Non - Inverting) Mode for OC0B pin:- void setup() { DDRD ¦= 32; // Set pin5 as Output TCCR0A = 32+2+1; // 32 for non-inverting mode PWM & 2+1 for Fast PWM generation // Non-inverting mode means clear OC0B on compare match and set OC0B at bottom TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0B = 25; // According to the value of OCR0B, output will change while(1){} } void loop() {} // Output > PWM output at OC0B pin (i.e., Arduino pin 5) |
| --- | --- |
| 10. Timer0 programming in Phase Correct PWM Mode (COM0A1=1 & COM0A0=1) for OC0A pin:- void setup() { DDRD ¦= 64; // Set pin6 as Output TCCR0A = 128+64+1; // 128+64 for set OC0A on compare match when up-counting // and clear OC0A on compare match when down-counting & 1 for Phase Correct PWM generation TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0A = 250; // According to the value of OCR0A, output will change while(1){} } void loop() {} // Output > PWM output at OC0A pin (i.e., Arduino pin 6) | 11. Timer0 programming in Phase Correct PWM Mode (COM0A1=1 & COM0A0=0) for OC0A pin:- void setup() { DDRD ¦= 64; // Set pin6 as Output TCCR0A = 128+1; // 128 for clear OC0A on compare match when up-counting // and set OC0A on compare match when down-counting & 1 for Phase Correct PWM generation TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0A = 10; // According to the value of OCR0A, output will change while(1){} } void loop() {} // Output > PWM output at OC0A pin (i.e., Arduino pin 6) |

| 12. Timer0 programming in Phase Correct PWM Mode (COM0B1=1 & COM0B0=1) for OC0B pin:- void setup() { DDRD ¦= 32; // Set pin5 as Output TCCR0A = 32+16+1; // 32+16 for set OC0B on compare match when up-counting // and clear OC0B on compare match when down-counting & 1 for Phase Correct PWM generation TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0B = 250; // According to the value of OCR0B, output will change while(1){} } void loop() {} // Output > PWM output at OC0B pin (i.e., Arduino pin 5) | 13. Timer0 programming in Phase Correct PWM Mode (COM0B1=1 & COM0B0=0) for OC0B pin:- void setup() { DDRD ¦= 32; // Set pin5 as Output TCCR0A = 32+1; // 32 for clear OC0B on compare match when up-counting // and set OC0B on compare match when down-counting & 1 for Phase Correct PWM generation TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0B = 10; // According to the value of OCR0B, output will change while(1){} } void loop() {} // Output > PWM output at OC0B pin (i.e., Arduino pin 5) |
| --- | --- |
| 14. Timer2 programming, Normal Mode, using Overflow Interrupt. (Timer0 Overflow Interrupt not working) :- void setup() { Serial.begin(9600); TCCR2A = 0; // Normal Mode TCCR2B = 5; // 5 for 1024 prescaler TIMSK2 = 1; // enable the Timer2 Overflow Interrupt TCNT2 = 0; while(1){} } ISR(TIMER2_OVF_vect){ Serial.println("hello world"); } void loop() {} | 15. Timer0 programming, CTC Mode Compare Match A:- void setup() { Serial.begin(9600); TCCR0A = 2; // CTC Mode TCCR0B = 5; // 5 for 1024 prescaler TIMSK0 = 2; // enable the Timer0 Output Compare Match A Interrupt TCNT0 = 0; OCR0A = 250; while(1){} } ISR(TIMER0_COMPA_vect){ Serial.println("hello world"); } void loop() {} |

| 16. Timer0 programming, CTC Mode Compare Match B:- void setup() { Serial.begin(9600); TCCR0A = 2; // CTC Mode TCCR0B = 5; // 5 for 1024 prescaler TIMSK0 = 4; //enable the Timer0 Output Compare Match B Interrupt TCNT0 = 0; OCR0A = 250; // if I use OCR0B = 250; then not working, may be it is the problem of the wokwi simulator while(1){} } ISR(TIMER0_COMPB_vect){ Serial.println("hello world"); } void loop() {} | 17. Use of TIFR Register (self) :- volatile long i=0; void setup() { Serial.begin(9600); TCCR0A = 2; // CTC Mode TCCR0B = 5; // 5 for 1024 prescaler TIMSK0 = 2; // enable the Timer0 Output Compare Match A Interrupt TCNT0 = 0; OCR0A = 250; while(1){ if(i%100==0) {Serial.println("hello world"); i++;} } } ISR(TIMER0_COMPA_vect){ if((TIFR0 & 2)==0) i++; } void loop() {} |
| --- | --- |
| 18. Brightness Control by using PWM Mode (self-written-code) :- volatile uint8_t count_flag, brightness=0; void setup() { DDRD ¦= 64; // Set pin6 as Output TCCR0A = 128+64+2+1; // 128+64 for inverting mode PWM & 2+1 for Fast PWM generation // Inverting mode means set OC0A on compare match and clear OC0A at bottom TCCR0B = 5; // 5 for 1024 prescaler TCNT0 = 0; OCR0A = 255; // According to the value of OCR0A, output will change TIMSK0 = 2; // Enable the Timer/Counter0 Output Compare Match A Interrupt while(1){} } ISR(TIMER0_COMPA_vect){ if(brightness==0) count_flag=0; if(brightness>250) count_flag=1; if(count_flag==0) brightness+=3; else brightness-=3; OCR0A = brightness; } void loop() {} | 19. Counter Programming by using Timer/Counter0 :- void setup() { Serial.begin(9600); DDRB = 1; TCCR0A = 0; // Normal Mode TCCR0B = 7; // 7 for selecting External Clock source on T0 pin (Rising Edge) TCNT0 = 0; while(1){ if(PORTB & 1) Serial.println(TCNT0); PORTB ^= 1; for(volatile long i=0; i<5000; i++); if(TCNT0>250) TCNT0=0; } } void loop() {} |

| 01. Timer1 programming in Normal Mode:- void setup() { DDRB = 32; // PORTB 5th bit set as Output TCCR1A = 0; // 0 for Normal Mode TCCR1B = 5; // 5 for selecting 1024 prescaler TCNT1 = 0; while(1){ if(TCNT1>=10000){ PORTB ^= 32; TCNT1=0; } } } void loop() {} |  |
| --- | --- |
| 02. Timer1 programming, Normal Mode, using Overflow Interrupt :- void setup() { DDRB = 32; // PORTB 5th bit set as Output TCCR1A = 0; // 0 for Normal Mode TCCR1B = 3; // 3 for selecting 64 prescaler TIMSK1 = 1; // for setting the Overflow Flag TCNT1 = 0; while(1){} } ISR(TIMER1_OVF_vect){ PORTB ^= 32; } void loop() {} |  |

Timer1 (16-bit Timer/Counter) Programming

| 03. Timer1 programming in CTC Mode, OC1A pin toggling on compare match:- void setup() { DDRB = 2; // PORTB 1st bit set as Output TCCR1A = 64; // 64 for toggling OC1A pin on compare match TCCR1B = 5 + 8; // 5 for selecting 1024 prescaler & 8 for selecting CTC Mode OCR1A = 5000; TCNT1 = 0; while(1){} } void loop() {} | For OC1A Pin Operation |
| --- | --- |
| 04. Timer1 programming in CTC Mode, OC1B pin toggling on compare match:- void setup() { DDRB = 4; // PORTB 2nd bit set as Output TCCR1A = 16; // 16 for toggling OC1B pin on compare match TCCR1B = 5 + 8; // 5 for selecting 1024 prescaler & 8 for selecting CTC Mode OCR1A = 5000; TCNT1 = 0; while(1){} } void loop() {} | For OC1B Pin Operation |

| 05. Timer1 programming in Fast PWM (10-bit & Inverting) Mode for OC1A pin:- void setup() { DDRB = 2; // PORTB 1st bit set as Output TCCR1A = 128 + 64 + 2 + 1; // 128 + 64 for Set OC1A on compare match, //clear OC1A at BOTTOM (inverting mode) // and 2 + 1 for selecting Fast PWM, 10-bit TCCR1B = 4 + 8; // 4 for selecting 256 prescaler & 8 for selecting Fast PWM, 10-bit OCR1A = 1000; TCNT1 = 0; while(1){} } void loop() {} | 06. Timer1 programming in Fast PWM (10-bit Non - Inverting) Mode for OC1A pin:- void setup() { DDRB = 2; // PORTB 1st bit set as Output TCCR1A = 128 + 2 + 1; // 128 for clear OC1A on compare match, //set OC1A at BOTTOM (Non-inverting mode) // and 2 + 1 for selecting Fast PWM, 10-bit TCCR1B = 4 + 8; // 4 for selecting 256 prescaler & 8 for selecting Fast PWM, 10-bit OCR1A = 10; TCNT1 = 0; while(1){} } void loop() {} |
| --- | --- |
| 07. Timer1 programming in Fast PWM (10-bit Inverting) Mode for OC1B pin:- void setup() { DDRB = 4; // PORTB 2nd bit set as Output TCCR1A = 32 + 16 + 2 + 1; // 32 + 16 for set OC1B on compare match, clear OC1B at BOTTOM (inverting mode) // and 2 + 1 for selecting Fast PWM, 10-bit TCCR1B = 4 + 8; // 4 for selecting 256 prescaler & 8 for selecting Fast PWM, 10-bit OCR1B = 1000; TCNT1 = 0; while(1){} } void loop() {} | 08. Timer1 programming in Fast PWM (10-bit Non - Inverting) Mode for OC1B pin:- void setup() { DDRB = 4; // PORTB 2nd bit set as Output TCCR1A = 32 + 2 + 1; // 32 for clear OC1B on compare match, set OC1B at BOTTOM (Non-inverting mode) // and 2 + 1 for selecting Fast PWM, 10-bit TCCR1B = 4 + 8; // 4 for selecting 256 prescaler & 8 for selecting Fast PWM, 10-bit OCR1B = 10; TCNT1 = 0; while(1){} } void loop() {} |

| 09. The High-lighted Modes are not done for the confusion :- | 10. Timer1 programming in Phase Correct PWM Mode [10-bit] (COM0A1=1 & COM0A0=1) for OC1A pin:- void setup() { DDRB = 2; // PORTB 1st bit set as Output TCCR1A = 128 + 64 + 2 + 1; // 128 + 64 for set OC1A on compare match when up-counting // & clear OC1A on compare match when down counting. // and 2 + 1 for selecting PWM, phase correct, 10-bit TCCR1B = 4 ; // 4 for selecting 256 prescaler OCR1A = 1000; TCNT1 = 0; while(1){} } void loop() {} |
| --- | --- |
| 11. Timer1 programming in Phase Correct PWM Mode [10-bit] (COM0A1=1 & COM0A0=0) for OC1A pin:- void setup() { DDRB = 2; // PORTB 1st bit set as Output TCCR1A = 128 + 2 + 1; // 128 for clear OC1A on compare match when up-counting // & set OC1A on compare match when down counting. // and 2 + 1 for selecting PWM, phase correct, 10-bit TCCR1B = 4 ; // 4 for selecting 256 prescaler OCR1A = 10; TCNT1 = 0; while(1){} } void loop() {} | 12. Timer1 programming in Phase Correct PWM Mode [10-bit] (COM1B1=1 & COM1B0=1) for OC1B pin:- void setup() { DDRB = 4; // PORTB 2nd bit set as Output TCCR1A = 32 + 16 + 2 + 1; // 32 + 16 for set OC1B on compare match when up-counting // & clear OC1B on compare match when down counting. // and 2 + 1 for selecting PWM, phase correct, 10-bit TCCR1B = 4 ; // 4 for selecting 256 prescaler OCR1B = 1000; TCNT1 = 0; while(1){} } void loop() {} |

| 13. Timer1 programming in Phase Correct PWM Mode [10-bit] (COM1B1=1 & COM1B0=0) for OC1B pin:- void setup() { DDRB = 4; // PORTB 2nd bit set as Output TCCR1A = 32 + 2 + 1; // 32 for clear OC1B on compare match when up-counting // & set OC1B on compare match when down counting. // and 2 + 1 for selecting PWM, phase correct, 10-bit TCCR1B = 4 ; // 4 for selecting 256 prescaler OCR1B = 10; TCNT1 = 0; while(1){} } void loop() {} | 14. Timer1 programming, CTC Mode Compare Match A:- void setup() { DDRB = 2; // PORTB 1st bit set as Output TCCR1A = 0; // TCCR1A = 0 & TCCR1B = 8 for CTC Mode TCCR1B = 8 + 4; // 4 for 256 prescaler TIMSK1 = 2; // for selecting the Output Compare Match A Interrupt OCR1A = 60000; TCNT1 = 0; while(1){} } ISR(TIMER1_COMPA_vect){ PORTB ^= 2; } void loop() {} |
| --- | --- |
| 15. Timer1 programming, CTC Mode Compare Match B:- void setup() { DDRB = 2; // PORTB 1st bit set as Output TCCR1A = 0; // TCCR1A = 0 & TCCR1B = 8 for CTC Mode TCCR1B = 8 + 4; // 4 for 256 prescaler TIMSK1 = 4; // for selecting the Output Compare Match B Interrupt OCR1A = 60000; TCNT1 = 0; while(1){} } ISR(TIMER1_COMPB_vect){ PORTB ^= 2; } void loop() {} | 16. Use of TIFR Register (self) :- void setup() { DDRB = 2; // PORTB 1st bit set as Output TCCR1A = 0; // TCCR1A = 0 & TCCR1B = 8 for CTC Mode TCCR1B = 8 + 4; // 4 for 256 prescaler TIMSK1 = 2; // for selecting the Output Compare Match A Interrupt OCR1A = 60000; TCNT1 = 0; while(1){} } ISR(TIMER1_COMPA_vect){ if((TIFR1 & 2)==0) PORTB ^= 2; } void loop() {} |