| 01. Master in Read Mode (only one byte) :- /* Master in Read Mode*/ #include<avr/io.h> void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); // polling e.g checking TWINT flag to check whether START condition transmitted succesfully } unsigned char i2c_read(unsigned char last){ if ( last == 0 ) // If want to read more than 1 byte TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); else // else read only one byte TWCR = (1<<TWINT) ¦ (1<<TWEN); while ((TWCR & ( 1 << TWINT ) ) == 0); return TWDR ; } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } int main(void){ unsigned char i = 0 ; DDRD = 0xFF; // PORTD is output i2c_init(); // initialse TWI for master mode i2c_start(); // transmit start condition i2c_write(0b11010001); // transmitt Slave address + R(1) (SLA+R means to master is reading ) i = i2c_read(1); // read only 1 byte PORTD= i; // show byte in PORTD i2c_stop(); // transmitt stop condition while(1); return 0; } | 02. Master in Write Mode (only one byte) :- /* Master in Write Mode*/ #include<avr/io.h> void i2c_init (void) { TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void) { TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data) { TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); // polling e.g checking TWINT flag to check whether START condition transmitted succesfully } void i2c_stop() { TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } int main(void) { i2c_init(); // initialse TWI for master mode i2c_start(); // transmit start condition i2c_write(0b11010000); // transmitt Slave address + W0 (SLA+W means to master is writing ) i2c_write(0b10101010); // transmitt data i2c_stop(); // transmitt start condition while(1); return 0; } |
| --- | --- |

-: I2C Protocol :-

| 03. Slave in Read Mode (only one byte) :- /* Slave in Read mode*/ #include<avr/io.h> void i2c_initslave (unsigned char slaveaddress) { TWCR = 0x04; // enable TWI module by setting TWEN flag TWAR = slaveaddress; // set slave address TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); // init TWI module } void i2c_listen() { while ((TWCR & ( 1 << TWINT ) ) == 0); // wait to be addressed by master } unsigned char i2c_read(unsigned char last) { if ( last == 0 ) // If want to read more than 1 byte TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); else // else read only one byte TWCR = (1<<TWINT) ¦ (1<<TWEN); while ((TWCR & ( 1 << TWINT ) ) == 0); return TWDR ; } int main(void) { DDRD = 0xFF; // PORTD is output i2c_initslave(0b11010001); // initiate TWI module as slave with address = 0b11010001 i2c_listen(); // listen to be addressed PORTD = i2c_read(1); // read only 1 byte while(1); return 0; } | 04. Slave in Write Mode (only one byte) :- /* Slave in write mode*/ #include<avr/io.h> void i2c_initslave (unsigned char slaveaddress) { TWCR = 0x04; // enable TWI module by setting TWEN flag TWAR = slaveaddress; // set slave address TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); // init TWI module } void i2c_listen() { while ((TWCR & ( 1 << TWINT ) ) == 0); // wait to be addressed by master } void i2c_send(unsigned char data) { TWDR = data ; // copy data to TWDR TWCR = ( 1 << TWINT ) ¦ ( 1 << TWEN ); // start transmission while ((TWCR & ( 1 << TWINT ) ) == 0) ; // wait to complete } int main(void) { i2c_initslave(0b11010001); // initiate TWI module as slave with address = 0b11010001 i2c_listen(); // listen to be addressed i2c_send('U'); //transmitt letter while(1); // stay here forever return 0; } |
| --- | --- |

| 05. Master in Read Mode (one byte at a time) :- /* Master in Read Mode*/ #include<avr/io.h> void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); // polling e.g checking TWINT flag to check whether START condition transmitted succesfully } unsigned char i2c_read(unsigned char last){ if ( last == 0 ) // If want to read more than 1 byte TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); else // else read only one byte TWCR = (1<<TWINT) ¦ (1<<TWEN); while ((TWCR & ( 1 << TWINT ) ) == 0); return TWDR ; } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } int main(void){ unsigned char i = 0 ; DDRD = 0xFF; // PORTD is output i2c_init(); // initialse TWI for master mode L1: i2c_start(); // transmit start condition i2c_write(0b11010001); // transmitt Slave address + R(1) (SLA+R means to master is reading ) i = i2c_read(0); // 0 for read multiple bytes PORTD= i; // show byte in PORTD i2c_stop(); // transmitt stop condition for(volatile unsigned long i=0; i<100000; i++); goto L1; while(1); return 0; } | 06. Master in Write Mode (one byte at a time) :- /* Master in Write Mode*/ #include<avr/io.h> void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); // polling e.g checking TWINT flag to check whether START condition transmitted succesfully } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } int main(void){ volatile uint8_t shift=0; i2c_init(); // initialise TWI for master mode L1: i2c_start(); // transmit start i2c_write(0b11010000); // transmitt Slave address + W0 (SLA+W means to master is writing ) i2c_write(1<<shift); // transmitt data i2c_stop(); // transmitt start condition for(volatile long i=0; i<100000; i++); shift++; if(shift==8) shift=0; goto L1; while(1); return 0; } |
| --- | --- |

| 07. Slave in Read Mode (one byte at a time) :- /* Slave in Read mode*/ #include<avr/io.h> void i2c_initslave (unsigned char slaveaddress) { TWCR = 0x04; // enable TWI module by setting TWEN flag TWAR = slaveaddress; // set slave address TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); // init TWI module } void i2c_listen() { while ((TWCR & ( 1 << TWINT ) ) == 0); // wait to be addressed by master } unsigned char i2c_read(unsigned char last) { if ( last == 0 ) // If want to read more than 1 byte TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); else // else read only one byte TWCR = (1<<TWINT) ¦ (1<<TWEN); while ((TWCR & ( 1 << TWINT ) ) == 0); return TWDR ; } int main(void) { DDRD = 0xFF; // PORTD is output i2c_initslave(0b11010001); // initiate TWI module as slave with address = 0b11010001 L1: i2c_listen(); // listen to be addressed PORTD = i2c_read(0); // 0 for read multiple bytes goto L1; while(1); return 0; } | 08. Slave in Write Mode (one byte at a time) :- /* Slave in write mode*/ #include<avr/io.h> volatile uint8_t shift=0; void i2c_initslave (unsigned char slaveaddress) { TWCR = 0x04; // enable TWI module by setting TWEN flag TWAR = slaveaddress; // set slave address TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); // init TWI module } void i2c_listen() { while ((TWCR & ( 1 << TWINT ) ) == 0); // wait to be addressed by master } void i2c_send(unsigned char data) { TWDR = data ; // copy data to TWDR TWCR = ( 1 << TWINT ) ¦ ( 1 << TWEN ); // start transmission while ((TWCR & ( 1 << TWINT ) ) == 0) ; // wait to complete } int main(void) { L1: i2c_initslave(0b11010001); // initiate TWI module as slave with address = 0b11010001 i2c_listen(); // listen to be addressed i2c_send(3<<shift); //transmitt letter shift++; if(shift==8) shift=0; goto L1; while(1); // stay here forever return 0; } |
| --- | --- |

| 09. Master in Write Mode (Multiple bytes) :- /* Master in Write Mode*/ #include<avr/io.h> void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); // polling e.g checking TWINT flag to check whether START condition transmitted succesfully } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } int main(void){ volatile uint8_t shift=0; i2c_init(); // initialise TWI for master mode L1: i2c_start(); // transmit start i2c_write(0b11010000); // transmitt Slave address + W0 (SLA+W means to master is writing ) i2c_write(1<<shift); // transmitt data i2c_write(7<<shift); // transmitt data i2c_stop(); // transmitt start condition for(volatile long i=0; i<100000; i++); shift++; if(shift==8) shift=0; goto L1; while(1); return 0; } | 10. Slave in Read Mode (Multiple bytes) :- /* Slave in Read mode*/ #include<avr/io.h> void i2c_initslave (unsigned char slaveaddress) { TWCR = 0x04; // enable TWI module by setting TWEN flag TWAR = slaveaddress; // set slave address TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); // init TWI module } void i2c_listen() { while ((TWCR & ( 1 << TWINT ) ) == 0); // wait to be addressed by master } unsigned char i2c_read(unsigned char last) { if ( last == 0 ) // If want to read more than 1 byte TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); else // else read only one byte TWCR = (1<<TWINT) ¦ (1<<TWEN); while ((TWCR & ( 1 << TWINT ) ) == 0); return TWDR ; } int main(void) { DDRD = 0xFF; // PORTD is output i2c_initslave(0b11010001); // initiate TWI module as slave with address = 0b11010001 L1: i2c_listen(); // listen to be addressed PORTD = i2c_read(0); // 0 for read multiple bytes goto L1; while(1); return 0; } |
| --- | --- |

| 11. 16*2 LCD in 4 bit mode (Proteus) :- #define command_high 4 #define command_low 0 #define data_high 5 #define data_low 1 #define slave_address 0x70 void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); // polling e.g checking TWINT flag to check whether START condition transmitted succesfully } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } ///////// ABOVE ARE THE I2C FUNCTIONS ///////////////// void lcd_init(){ send_command (0x02); // 4bit mode //send_command (0x28); // Initialization of 16X2 LCD in 4bit mode send_command (0x0E); // Display ON Cursor ON send_command (0x06); // Auto Increment cursor send_command (0x01); // Clear display send_command (0x80); // Cursor at home position } void send_command(volatile uint8_t cmd){ volatile uint8_t high_nibble = cmd & 0xf0; volatile uint8_t low_nibble = cmd << 4; i2c_start(); // transmit start i2c_write(slave_address); // transmitt Slave address + W0 (SLA+W means to master is writing ) i2c_write(high_nibble + command_high); i2c_write(high_nibble + command_low); i2c_write(low_nibble + command_high); i2c_write(low_nibble + command_low); i2c_stop(); // transmitt stop condition delayMicroseconds(500); } | void send_data(volatile uint8_t data1){ volatile uint8_t high_nibble = data1 & 0xf0; volatile uint8_t low_nibble = data1 << 4; i2c_start(); // transmit start i2c_write(slave_address); // transmitt Slave address + W0 (SLA+W means to master is writing ) i2c_write(high_nibble + data_high); i2c_write(high_nibble + data_low); i2c_write(low_nibble + data_high); i2c_write(low_nibble + data_low); i2c_stop(); // transmitt stop condition delayMicroseconds(50); } void lcd_string (char *str) /* Send string to LCD function */ { int i; for(i=0;str[i]!=0;i++) /* Send each char of string till the NULL */ { send_data (str[i]); /* Call LCD data write */ } } int main(void){ i2c_init(); // initialise TWI for master mode lcd_init(); // initialise the 16*2 LCD lcd_string("Hello World!"); while(1); return 0; } The only difference between wokwi and Proteus for this program is the delay(); function and its value |
| --- | --- |

| 12. 16*2 LCD in 4 bit mode (Wokwi) :- /* Note: in this program or any other program does not use the delay() function,if delay() function is used, then overall program does not work, instead of it, use delayMicrosecond() */ void LCD_Init (void) /* LCD Initialize function */ { delayMicroseconds(2000); /* LCD Power ON Initialization time >15ms */ LCD_Command (0x02); // 4bit mode LCD_Command (0x28); // 4bit mode LCD_Command (0x0C); //Display ON Cursor OFF LCD_Command (0x06); // Auto Increment cursor LCD_Command (0x01); // Clear display LCD_Command (0x80); // Cursor at home position } void LCD_Command (volatile uint8_t cmnd) { PORTB = cmnd >> 4; /* Send upper nibble */ PORTD = 4; delayMicroseconds(200); PORTD = 0; delayMicroseconds(200); PORTB = cmnd & 0x0f;/* Send lower nibble */ PORTD = 4; delayMicroseconds(200); PORTD = 0; delayMicroseconds(200); } void LCD_Char (volatile uint8_t data) { PORTB = data >> 4; /* Send upper nibble */ PORTD = 8 + 4; delayMicroseconds(200); PORTD = 8; delayMicroseconds(200); PORTB = data & 0x0f;/* Send lower nibble */ PORTD = 8 + 4; delayMicroseconds(200); PORTD = 8; delayMicroseconds(200); } | void LCD_String (volatile uint8_t *str) /* Send string to LCD function */ { int i; for(i=0;str[i]!=0;i++) /* Send each volatile uint8_t of string till the NULL */ { LCD_Char (str[i]); /* Call LCD data write */ } } int main() { DDRD = 0x0c; DDRB = 0x0f; LCD_Init(); /* Initialization of LCD*/ LCD_String("Hello World!"); /* write string on 1st line of LCD*/ while(1); /* Infinite loop. */ return 0; } |
| --- | --- |

| 13. 16*2 LCD through I2C Protocol (Wokwi) :- #define command_en_high 4 + 8 // RS=0, R/W=0, EN=1 (4), LED+ = 1 (8) #define command_en_low 0 + 8 // RS=0, R/W=0, EN=0, LED+ = 1 (8) #define data_en_high 1 + 4 + 8 // RS=1 (1), R/W=0, EN=1 (4), LED+ = 1 (8) #define data_en_low 1 + 8 // RS=1 (1), R/W=0, EN=0, LED+ = 1 (8) #define slave_address 0x27 << 1 // slave address of the I2C LCD void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } ///////////// ABOVE ARE THE I2C FUNCTIONS ////////////// ///////////// BELOW ARE THE LCD FUNCTIONS ///////////// void lcd_init(){ send_command (0x02); // 4bit mode //send_command (0x28); // 4bit mode send_command (0x0C); // Display ON Cursor OFF send_command (0x06); // Auto Increment cursor send_command (0x01); // Clear display send_command (0x80); // Cursor at home position } void send_command(volatile uint8_t cmd){ volatile uint8_t high_nibble = cmd & 0xf0; volatile uint8_t low_nibble = cmd << 4; i2c_start(); // transmit start i2c_write(slave_address); // transmitt Slave address + W0 (SLA+W means to master is writing ) | i2c_write(high_nibble + command_en_high); i2c_write(high_nibble + command_en_low); i2c_write(low_nibble + command_en_high); i2c_write(low_nibble + command_en_low); i2c_stop(); // transmit stop condition delayMicroseconds(50); } void send_data(volatile uint8_t data1){ volatile uint8_t high_nibble = data1 & 0xf0; volatile uint8_t low_nibble = data1 << 4; i2c_start(); // transmit start i2c_write(slave_address); // transmitt Slave address + W0 (SLA+W means to master is writing ) i2c_write(high_nibble + data_en_high); i2c_write(high_nibble + data_en_low); i2c_write(low_nibble + data_en_high); i2c_write(low_nibble + data_en_low); i2c_stop(); // transmitt stop condition delayMicroseconds(50); } void lcd_string (char *str){ int i; for(i=0;str[i]!=0;i++) send_data (str[i]); /* Call LCD data write */ } int main(void){ i2c_init(); // initialise TWI for master mode lcd_init(); // initialise the 16*2 LCD lcd_string("Hello World!"); while(1); return 0; } |
| --- | --- |

| 14. I2C Read & Write Program :- #include<avr/io.h> void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); } unsigned char i2c_read(unsigned char last){ if ( last == 0 ) // If want to read more than 1 byte TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); else // else read only one byte TWCR = (1<<TWINT) ¦ (1<<TWEN); while ((TWCR & ( 1 << TWINT ) ) == 0); return TWDR ; } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } int main(void){ volatile uint8_t rcvData=0; while(1){ i2c_start(); i2c_write(0x40); // slave address + write i2c_write(rcvData¦0x0F); // Write lower nibble to HIGH and higher nibble to rcvData i2c_stop(); i2c_start(); i2c_write(0x41); // slave address + read rcvData=i2c_read(1); // Read the input of lower nibble i2c_stop(); rcvData <<= 4; //Shift the inputs right to the output rcvData = ~rcvData; delayMicroseconds(100); } return 0; } | Below figure only for the I2C LCD |
| --- | --- |

| 15. I2C RTC Program (Serial Print) :- #include<avr/io.h> /* Page - 1 */ void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); } unsigned char i2c_read(unsigned char last){ if ( last == 0 ) // If want to read more than 1 byte TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); else // else read only one byte TWCR = (1<<TWINT) ¦ (1<<TWEN); while ((TWCR & ( 1 << TWINT ) ) == 0); return TWDR ; } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } //----------------------------------------------------- volatile uint8_t read_second(){ volatile uint8_t sec, second_ones, second_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x00); // second address 0x00 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read sec = i2c_read(1); i2c_stop(); second_ones = 0x0f & sec; second_tens = 0x07 & (sec >> 4); sec = (second_tens * 10) + second_ones; // calculating the atual second value return sec; } i2c_start(); /* Page - 3 */ i2c_write(0b11010000); // slave address + write i2c_write(0x04); // date address 0x04 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read date = i2c_read(1); i2c_stop(); date_ones = 0x0f & date; date_tens = 0x03 & (date >> 4); date = (date_tens * 10) + date_ones; // calculating the atual date value return date; } volatile uint8_t read_month(){ volatile uint8_t month, month_ones, month_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x05); // month address 0x05 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read month = i2c_read(1); i2c_stop(); month_ones = 0x0f & month; month_tens = 0x01 & (month >> 4); month = (month_tens * 10) + month_ones; // calculating the atual month value return month; } volatile uint8_t read_year(){ volatile uint8_t year, year_ones, year_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x06); // year address 0x06 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read year = i2c_read(1); i2c_stop(); year_ones = 0x0f & year; year_tens = year >> 4; year = (year_tens * 10) + year_ones; // calculating the atual year value return year; } int main(){ Serial.begin(9600); volatile uint8_t seconds, minutes, hours, days, dates, months, years; | volatile uint8_t read_minute(){ /* Page - 2 */ volatile uint8_t minute, minute_ones, minute_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x01); // minute address 0x01 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read minute = i2c_read(1); i2c_stop(); minute_ones = 0x0f & minute; minute_tens = minute >> 4; minute = (minute_tens * 10) + minute_ones; // calculating the atual minute value return minute; } volatile uint8_t read_hour(){ volatile uint8_t hour, hour_ones, hour_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x02); // hour address 0x02 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read hour = i2c_read(1); i2c_stop(); hour_ones = 0x0f & hour; hour_tens = 0x03 & (hour >> 4); hour = (hour_tens * 10) + hour_ones; // calculating the atual hour value return hour; } volatile uint8_t read_day(){ volatile uint8_t day; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x03); // day address 0x03 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read day = i2c_read(1); i2c_stop(); day &= 0x07; // calculating the atual day value return day; } volatile uint8_t read_date(){ volatile uint8_t date, date_ones, date_tens; char day[][12] = {"Saturday", "Sunday", "Monday", "Tuesday", "Wednessday", "Thursday", "Friday"}; i2c_init(); /* Page - 4 */ while(1){ Serial.print("TIME:- "); // printing the time // hours = read_hour(); Serial.print(hours); Serial.print(':'); minutes = read_minute(); Serial.print(minutes); Serial.print(':'); seconds = read_second(); Serial.print(seconds); Serial.println(); Serial.print("DATE:- "); // printing the date // dates = read_date(); Serial.print(dates); Serial.print('/'); months = read_month(); Serial.print(months); Serial.print('/'); years = read_year(); Serial.print("20"); Serial.print(years); Serial.println(); Serial.print("DAY:- "); // printing the day // days = read_day(); Serial.print(day[days]); Serial.println(); Serial.println(); Serial.println(); for(volatile long i=0; i<100000; i++); // delay // } return 0; } |
| --- | --- |

| 16. I2C RTC with I2C 16*2 LCD Program :- #include "I2C.h" // Page 1 // #include "LCD1602.h" #include "RTC1307.h" int main(){ volatile uint8_t seconds, minutes, hours, days, dates, months, years; char day[][12] = {" Sunday", " Monday", " Tuesday", " Wednessday", " Thursday", " Friday", " Saturday",}; i2c_init(); lcd_init(); while(1){ for(volatile long i=0; i<300; i++){ // printing the Time & Date for a particular time // lcd_cursor(0,0); lcd_string("TIME-"); hours = read_hour(); // for reading the hour lcd_number(hours); // for printing the hour lcd_string(":"); minutes = read_minute(); // for reading the minute lcd_number(minutes); // for printing the minute lcd_string(":"); seconds = read_second(); // for reading the second lcd_number(seconds); // for printing the second lcd_string(" "); lcd_cursor(1,0); lcd_string("DATE-"); dates = read_date(); // for reading the date lcd_number(dates); // for printing the date lcd_string("/"); months = read_month(); // for reading the month lcd_number(months); // for printing the month lcd_string("/"); years = read_year(); // for reading the year lcd_number(20); lcd_number(years); // for printing the year lcd_string(" "); } lcd_clear(); lcd_cursor(0,0); lcd_string("Today is:-"); lcd_cursor(1,0); days = read_day(); // for reading the day lcd_string(day[days - 1]); // for printing the day lcd_string(" "); for(volatile long i=0; i<1000000; i++); // delay // } return 0; } | # I2C.h (header file) # #include<avr/io.h> // Page 2 // void i2c_init (void){ TWSR = 0X00; // set prescaler bits to zero TWBR = 0x47; // SCL frequency is 50K for XTAL = 8Mhz TWCR = 0x04; // enable TWI module } void i2c_start(void){ TWCR = ( 1 << TWINT ) ¦ ( 1 << TWSTA ) ¦ ( 1 << TWEN ); while ((TWCR & ( 1 << TWINT ) ) == 0) ; } void i2c_write(unsigned char data){ TWDR = data ; TWCR = (1<<TWINT) ¦ (1<<TWEN); // enable the transmission while ((TWCR & ( 1 << TWINT ) ) == 0); } unsigned char i2c_read(unsigned char last){ if ( last == 0 ) // If want to read more than 1 byte TWCR = (1<<TWINT) ¦ (1<<TWEN) ¦ (1<<TWEA); else // else read only one byte TWCR = (1<<TWINT) ¦ (1<<TWEN); while ((TWCR & ( 1 << TWINT ) ) == 0); return TWDR ; } void i2c_stop(){ TWCR = ( 1 <<TWINT )¦( 1 << TWEN) ¦ ( 1<< TWSTO); } |
| --- | --- |

| # RTC1307.h (header file) # volatile uint8_t read_second(){ // Page 3 // volatile uint8_t sec, second_ones, second_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x00); // second address 0x00 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read sec = i2c_read(1); i2c_stop(); second_ones = 0x0f & sec; second_tens = 0x07 & (sec >> 4); sec = (second_tens * 10) + second_ones; // calculating the actual second value return sec; } volatile uint8_t read_minute(){ volatile uint8_t minute, minute_ones, minute_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x01); // minute address 0x01 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read minute = i2c_read(1); i2c_stop(); minute_ones = 0x0f & minute; minute_tens = minute >> 4; minute = (minute_tens * 10) + minute_ones; // calculating the actual minute value return minute; } volatile uint8_t read_hour(){ volatile uint8_t hour, hour_ones, hour_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x02); // hour address 0x02 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read hour = i2c_read(1); i2c_stop(); hour_ones = 0x0f & hour; hour_tens = 0x03 & (hour >> 4); hour = (hour_tens * 10) + hour_ones; // calculating the actual hour value return hour; } volatile uint8_t read_day(){ volatile uint8_t day; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x03); // day address 0x03 i2c_start(); // repeated start | i2c_write(0b11010001); // slave address + read day = i2c_read(1); i2c_stop(); // Page 4 // day &= 0x07; // calculating the actual day value return day; } volatile uint8_t read_date(){ volatile uint8_t date, date_ones, date_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x04); // date address 0x04 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read date = i2c_read(1); i2c_stop(); date_ones = 0x0f & date; date_tens = 0x03 & (date >> 4); date = (date_tens * 10) + date_ones; // calculating the actual date value return date; } volatile uint8_t read_month(){ volatile uint8_t month, month_ones, month_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x05); // month address 0x05 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read month = i2c_read(1); i2c_stop(); month_ones = 0x0f & month; month_tens = 0x01 & (month >> 4); month = (month_tens * 10) + month_ones; // calculating the actual month value return month; } volatile uint8_t read_year(){ volatile uint8_t year, year_ones, year_tens; i2c_start(); i2c_write(0b11010000); // slave address + write i2c_write(0x06); // year address 0x06 i2c_start(); // repeated start i2c_write(0b11010001); // slave address + read year = i2c_read(1); i2c_stop(); year_ones = 0x0f & year; year_tens = year >> 4; year = (year_tens * 10) + year_ones; // calculating the actual year value return year; } |
| --- | --- |

| # lcd1602.h (header file) # void send_command(volatile uint8_t cmd); // Page 5 // #define command_en_high 4 + 8 // RS=0, R/W=0, EN=1 (4), LED+ = 1 (8) #define command_en_low 0 + 8 // RS=0, R/W=0, EN=0, LED+ = 1 (8) #define data_en_high 1 + 4 + 8 // RS=1 (1), R/W=0, EN=1 (4), LED+ = 1 (8) #define data_en_low 1 + 8 // RS=1 (1), R/W=0, EN=0, LED+ = 1 (8) #define slave_address 0x27 << 1 // slave address of the I2C LCD void lcd_init(){ send_command (0x02); // 4bit mode //send_command (0x28); // 4bit mode send_command (0x0C); // Display ON Cursor OFF send_command (0x06); // Auto Increment cursor send_command (0x01); // Clear display send_command (0x80); // Cursor at home position } void send_command(volatile uint8_t cmd){ volatile uint8_t high_nibble = cmd & 0xf0; volatile uint8_t low_nibble = cmd << 4; i2c_start(); // transmit start i2c_write(slave_address); // transmitt Slave address + W0 (SLA+W means to master is writing ) i2c_write(high_nibble + command_en_high); i2c_write(high_nibble + command_en_low); i2c_write(low_nibble + command_en_high); i2c_write(low_nibble + command_en_low); i2c_stop(); // transmit stop condition delayMicroseconds(50); } void send_data(volatile uint8_t data1){ volatile uint8_t high_nibble = data1 & 0xf0; volatile uint8_t low_nibble = data1 << 4; i2c_start(); // transmit start i2c_write(slave_address); // transmitt Slave address + W0 (SLA+W means to master is writing ) i2c_write(high_nibble + data_en_high); i2c_write(high_nibble + data_en_low); i2c_write(low_nibble + data_en_high); i2c_write(low_nibble + data_en_low); i2c_stop(); // transmitt stop condition delayMicroseconds(50); } | void lcd_string (char *str){ // Page 6 // int i; for(i=0;str[i]!=0;i++) send_data (str[i]); /* Call LCD data write */ } void lcd_number(volatile uint16_t number){ volatile char i=0, num[8]={0}; while(number){ num[i++] = number%10; number/=10; } i--; for(; i>=0; i--) send_data(num[i] + 48); } void lcd_cursor(volatile uint8_t row, volatile uint8_t column){ if(row==0) send_command(0x80 + column); else if(row==1) send_command(0xc0 + column); } void lcd_clear(){ send_command(0x01); } |
| --- | --- |

-: SPI Protocol :-

| 01. SPI General Program (Master) :- [CPOL=1, CPHA=1] void SPI_MasterInit(void){ DDRB = 0x2c; // SCK, MOSI, SS set as Output & MISO as Input PORTB ¦= 0x04; // Make SS pin High SPCR ¦= (1<<SPE) ¦ (1<<MSTR) ¦ (1<<CPOL) ¦ (1<<CPHA) ¦ (1<<SPR0); } void SPI_MasterTransmit(volatile uint8_t cData){ SPDR = cData; PORTB &= ~0x04; // Make SS pin Low while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed PORTB ¦= 0x04; // Make SS pin High delayMicroseconds(2); } void setup(){ SPI_MasterInit(); SPI_MasterTransmit(0x55); } void loop(){} | (Slave) :- void SPI_SlaveInit(void) { DDRB = 0x10; // MISO set as Output & SCK, MOSI, SS as Input SPCR ¦= (1<<SPE) ¦ (1<<CPOL) ¦ (1<<CPHA); } char SPI_SlaveReceive(void) { while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed return SPDR; } void setup(){ DDRD = 0xff; // Set Port D as Output SPI_SlaveInit(); PORTD = SPI_SlaveReceive(); } void loop(){} |
| --- | --- |

Only Master to Slave Transmission

(Half Duplex)

| 02. SPI General Program (Master) :- [CPOL=0, CPHA=0] void SPI_MasterInit(void){ DDRB = 0x2c; // SCK, MOSI, SS set as Output & MISO as Input PORTB ¦= 0x04; // Make SS pin High SPCR ¦= (1<<SPE) ¦ (1<<MSTR) ¦ (1<<SPR0); } void SPI_MasterTransmit(volatile uint8_t cData){ SPDR = cData; PORTB &= ~0x04; // Make SS pin Low while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed PORTB ¦= 0x04; // Make SS pin High delayMicroseconds(2); } void setup(){ SPI_MasterInit(); SPI_MasterTransmit(0x55); } void loop(){} | (Slave) :- void SPI_SlaveInit(void){ DDRB = 0x10; // MISO set as Output & SCK, MOSI, SS as Input SPCR ¦= (1<<SPE); } char SPI_SlaveReceive(void){ while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed return SPDR; } void setup(){ DDRD = 0xff; // Set Port D as Output SPI_SlaveInit(); PORTD = SPI_SlaveReceive(); } void loop(){} |
| --- | --- |
| 03. SPI LED Shift Program (Master) :- void SPI_MasterInit(void){ DDRB = 0x2c; // SCK, MOSI, SS set as Output PORTB ¦= 0x04; // Make SS pin High SPCR ¦= (1<<SPE) ¦ (1<<MSTR) ¦ (1<<SPR0); } void SPI_MasterTransmit(volatile uint8_t cData){ SPDR = cData; PORTB &= ~0x04; // Make SS pin Low while(!(SPSR & (1<<SPIF))); PORTB ¦= 0x04; // Make SS pin High delayMicroseconds(2); } void setup(){ volatile uint8_t shift=0; SPI_MasterInit(); while(1){ SPI_MasterTransmit(3<<shift); shift++; if(shift==8) shift=0; for(volatile long i=0; i<100000; i++); }} void loop(){} | (Slave) :- void SPI_SlaveInit(void){ DDRB = 0x10; // MISO set as Output & SCK, MOSI, SS as Input SPCR ¦= (1<<SPE); } char SPI_SlaveReceive(void){ while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed return SPDR; } void setup(){ DDRD = 0xff; // Set Port D as Output SPI_SlaveInit(); while(1) PORTD = SPI_SlaveReceive(); } void loop(){} |

| 04. SPI LED Program [Full Duplex] (Master) :- void SPI_MasterInit(void){ DDRB = 0x2c; // SCK, MOSI, SS set as Output & MISO as Input PORTB ¦= 0x04; // Make SS pin High SPCR ¦= (1<<SPE) ¦ (1<<MSTR) ¦ (1<<SPR0); } void SPI_MasterTransmit(volatile uint8_t cData){ SPDR = cData; PORTB &= ~0x04; // Make SS pin Low while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed PORTB ¦= 0x04; // Make SS pin High } char SPI_MasterReceive(void){ while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed return SPDR; } void setup(){ DDRD = 0xff; SPI_MasterInit(); while(1){ SPI_MasterTransmit(0x50); PORTD = SPI_MasterReceive(); } } void loop(){} | (Slave) :- void SPI_SlaveInit(void){ DDRB = 0x10; // MISO set as Output & SCK, MOSI, SS as Input SPCR ¦= (1<<SPE); } char SPI_SlaveReceive(void){ while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed return SPDR; } void SPI_SlaveTransmit(volatile uint8_t cData){ SPDR = cData; while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed //delayMicroseconds(2); } void setup(){ DDRD = 0xff; // Set Port D as Output SPI_SlaveInit(); while(1) { PORTD = SPI_SlaveReceive(); SPI_SlaveTransmit(0x03); } } void loop(){} |
| --- | --- |

Master to Slave & Vice-Versa (Full Duplex)

In real hardware, this program works fine but in Proteus, the Master receives wrong data, Ex – The Slave send data 0x03 but the Master receives 0x81 as shown in the figure.

Master to Slave & vice-versa

(Full Duplex)

| 05. SPI Keyboard & LCD Program (Master) :- //---------------------------------- SPI Functions -------------------------------// void SPI_MasterInit(void){ DDRB = 0x2c; // SCK, MOSI, SS set as Output & MISO as Input PORTB ¦= 0x04; // Make SS pin High SPCR ¦= (1<<SPE) ¦ (1<<MSTR) ¦ (1<<SPR0); } void SPI_MasterTransmit(volatile uint8_t cData){ SPDR = cData; PORTB &= ~0x04; // Make SS pin Low while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed PORTB ¦= 0x04; // Make SS pin High } char SPI_MasterReceive(void){ while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed return SPDR; } //------------------- KEYBOARD SCANNING FUNCTION ----------------------// volatile char checkKey(){ volatile char row, col, key; for(row=0; row<4; row++){ PORTC = 1 << row; if(PIND){ col=(PIND) >> 4; break;} } if(row==0){ if(col==1) key='7'; else if(col==2) key='8'; else if(col==4) key='9'; else if(col==8) key='/'; } else if(row==1){ if(col==1) key='4'; else if(col==2) key='5'; else if(col==4) key='6'; else if(col==8) key='*'; } else if(row==2){ if(col==1) key='1'; else if(col==2) key='2'; else if(col==4) key='3'; else if(col==8) key='-'; } | (Slave) :- //------------------------------- SPI Functions -----------------------------// void SPI_SlaveInit(void){ DDRB = 0x10; // MISO set as Output & SCK, MOSI, SS as Input SPCR ¦= (1<<SPE); } char SPI_SlaveReceive(void){ while(!(SPSR & (1<<SPIF))); // Wait until the Serial Transfer is completed return SPDR; } //------------------------------- LCD Functions ----------------------------// void LCD_Init (void) { LCD_Command (0x02); // 8bit mode LCD_Command (0x28); // 8bit mode LCD_Command (0x0C); //Display ON Cursor OFF LCD_Command (0x06); // Auto Increment cursor LCD_Command (0x01); // Clear display LCD_Command (0x80); // Cursor at home position delayMicroseconds(2000); } void LCD_Command (volatile uint8_t cmnd) { PORTD = (cmnd & 0xf0) + 4; /* Send upper nibble */ delayMicroseconds(50); PORTD &= ~4; delayMicroseconds(50); PORTD = (cmnd << 4) + 4; /* Send lower nibble */ delayMicroseconds(50); PORTD &= ~4; delayMicroseconds(50); } void LCD_Data (volatile uint8_t data) { PORTD = (data & 0xf0) + 12; /* Send upper nibble */ delayMicroseconds(50); PORTD &= ~4; delayMicroseconds(50); PORTD = (data << 4) + 12; /* Send lower nibble */ delayMicroseconds(50); PORTD &= ~4; delayMicroseconds(50); } |
| --- | --- |

| else if(row==3){ if(col==1) key='c'; else if(col==2) key='0'; else if(col==4) key='='; else if(col==8) key='+'; } else key=0; return key; } //----------------------------------- Main Function ----------------------------// void setup(){ DDRD = 0; DDRC = 0x0F; volatile char send_data; SPI_MasterInit(); while(1){ send_data = checkKey(); if(send_data) SPI_MasterTransmit(send_data); for(volatile long i=0; i<70000; i++); } } | //------------------------------ Main Function ----------------------------// void setup(){ DDRD = 0xff; DDRC = 0x0f; volatile uint8_t data; SPI_SlaveInit(); LCD_Init(); /* Initialization of LCD*/ while(1){ data = SPI_SlaveReceive(); if(data=='c') LCD_Command(0x01); // clear the display else LCD_Data(data); // print the data } } |
| --- | --- |

ON/C - for clear the display

-: USART Protocol :-

| /* 01. UART Transmitter Code (Sending ‘A’) */ void init_USART0(){ volatile uint16_t UBRR_value = 103; // UBBR0 = ((clock frequency/(16*baud rate)) - 1) // UBBR0 = ((16,000,000/(16*9600)) -1) = 103 UBRR0H = (unsigned char)(UBRR_value >> 8); UBRR0L = (unsigned char) UBRR_value; // Select the 9600 baud rate UCSR0B = (1 << RXEN0) ¦ (1 << TXEN0); // enable Tx and Rx UCSR0C = (1 << USBS0) ¦ (3 << UCSZ00); // set 2 stop bit & 8 data bit } void setup() { DDRD &= ~4; // set port-D 2nd pin as Input DDRB = 1; // set port-B 0th pin as Output init_USART0(); while(1){ if((PIND & 4)==0){ PORTB ^= 1; while(!(UCSR0A & (1 << UDRE0))); // wait untill the UDRE0 bit is set UDR0 = 65; // 65 = A for(volatile long i=0; i<100000; i++); // switch debouncing delay } } } void loop() {} | /* UART Receiver Code */ void init_USART0(){ volatile uint16_t UBRR_value = 103; // UBBR0 = ((clock frequency/(16*baud rate)) - 1) // UBBR0 = ((16,000,000/(16*9600)) -1) = 103 UBRR0H = (unsigned char)(UBRR_value >> 8); UBRR0L = (unsigned char) UBRR_value; // Select the 9600 baud rate UCSR0B = (1 << RXEN0) ¦ (1 << TXEN0); // enable Tx and Rx UCSR0C = (1 << USBS0) ¦ (3 << UCSZ00); // set 2 stop bit & 8 data bit } void setup() { DDRB = 1; // port-B 0th bit set as Output init_USART0(); volatile uint8_t receive_data; while(1){ while(!(UCSR0A & (1 << RXC0))); // wait until the RXC0 bit is set receive_data = UDR0; if(receive_data == 'A'){ PORTB ^= 1; } } } void loop() {} |
| --- | --- |

| /* 02. UART Transmit and Receive using Rx complete interrupt (Transmitter Code) */ volatile uint8_t receive_data; void init_USART0(){ volatile uint16_t UBRR_value = 103; // UBBR0 = ((clock frequency/(16*baud rate)) - 1) // UBBR0 = ((16,000,000/(16*9600)) -1) = 103 UBRR0H = (unsigned char)(UBRR_value >> 8); UBRR0L = (unsigned char) UBRR_value; // Select the 9600 baud rate UCSR0B = (1 << RXEN0) ¦ (1 << TXEN0) ¦ (1<<RXCIE0); // enable Tx, Rx and Rx complete interrupt UCSR0C = (1 << USBS0) ¦ (3 << UCSZ00); // set 2 stop bit & 8 data bit } void USART_send(volatile char data){ while(!(UCSR0A & (1 << UDRE0))); // wait untill the UDRE0 bit is set UDR0 = data; } void setup() { DDRD &= ~4; // set port-D 2nd pin as Input DDRB = 1; // set port-B 0th pin as Output init_USART0(); while(1){ if((PIND & 4)==0){ USART_send('A'); for(volatile long i=0; i<100000; i++); // switch debouncing delay } if(receive_data == 'B'){ PORTB ^= 1; receive_data=0; } } } ISR(USART_RX_vect){ receive_data = UDR0; } | /* Receiver Code */ volatile uint8_t receive_data; void init_USART0(){ volatile uint16_t UBRR_value = 103; // UBBR0 = ((clock frequency/(16*baud rate)) - 1) // UBBR0 = ((16,000,000/(16*9600)) -1) = 103 UBRR0H = (unsigned char)(UBRR_value >> 8); UBRR0L = (unsigned char) UBRR_value; // Select the 9600 baud rate UCSR0B = (1 << RXEN0) ¦ (1 << TXEN0) ¦ (1<<RXCIE0); // enable Tx, Rx and Rx complete interrupt UCSR0C = (1 << USBS0) ¦ (3 << UCSZ00); // set 2 stop bit & 8 data bit } void USART_send(volatile char data){ while(!(UCSR0A & (1 << UDRE0))); // wait untill the UDRE0 bit is set UDR0 = data; } void setup() { DDRD &= ~4; // set port-D 2nd pin as Input DDRB = 1; // port-B 0th bit set as Output init_USART0(); while(1){ if((PIND & 4)==0){ USART_send('B'); for(volatile long i=0; i<100000; i++); // switch debouncing delay } if(receive_data == 'A'){ PORTB ^= 1; receive_data=0; } } } ISR(USART_RX_vect){ receive_data = UDR0; } |
| --- | --- |

This figure is the circuit diagram of the above code

| /* UART Transmitter Code */ void init_USART0(){ volatile uint16_t UBRR_value = 103; // UBBR0 = ((clock frequency/(16*baud rate)) - 1) // UBBR0 = ((16,000,000/(16*9600)) -1) = 103 UBRR0H = (unsigned char)(UBRR_value >> 8); UBRR0L = (unsigned char) UBRR_value; // Select the 9600 baud rate UCSR0B = (1 << RXEN0) ¦ (1 << TXEN0); // enable Tx and Rx UCSR0C = (1 << USBS0) ¦ (3 << UCSZ00); // set 2 stop bit & 8 data bit } void setup() { DDRD &= ~4; // set port-D 2nd pin as Input DDRB = 1; // set port-B 0th pin as Output init_USART0(); while(1){ if((PIND & 4)==0){ PORTB ^= 1; while(!(UCSR0A & (1 << UDRE0))); // wait untill the UDRE0 bit is set UDR0 = 'A'; for(volatile long i=0; i<100000; i++); // switch debouncing delay } } } void loop() {} |  |
| --- | --- |

-: USART Protocol in SPI Mode :-

| /* Master Code (only Master generates the clock (in XCK pin)) [Simple LED Output] using USART Rx interrupt */ volatile char receive_data; void USART_Init(){ UBRR0 = 0; DDRD ¦= 16; // XCK pin (i.e - PD4) set as output UCSR0C = (1<<UMSEL01)¦(1<<UMSEL00)¦(1<<UCPHA0)¦(1<<UCPOL0); UCSR0B = (1<<RXEN0)¦(1<<TXEN0)¦(1<<RXCIE0); // Enable Tx, Rx & Recive complete interrupt UBRR0 = 832; // set baud rate as 9600 } void USART_Transmit(volatile char data){ while (!(UCSR0A & (1<<UDRE0))); // Wait for empty transmit buffer UDR0 = data; } void setup(){ DDRB = 0x0f; USART_Init(); while(1){ USART_Transmit(0x0C); if(receive_data){ PORTB = receive_data; receive_data = 0; } } } ISR(USART_RX_vect){ receive_data = UDR0; } | /* Slave Code [Simple LED Output]*/ volatile char receive_data; void USART_Init(){ UBRR0 = 0; DDRD &= ~16; // XCK pin (i.e - PD4) set as Input as slave UCSR0C = (1<<UMSEL01)¦(1<<UMSEL00)¦(1<<UCPHA0)¦(1<<UCPOL0); UCSR0B = (1<<RXEN0)¦(1<<TXEN0)¦(1<<RXCIE0); // Enable Tx, Rx & Recive complete interrupt UBRR0 = 832; // set baud rate as 9600 } void USART_Transmit(volatile char data){ while (!(UCSR0A & (1<<UDRE0))); // Wait for empty transmit buffer UDR0 = data; } void setup(){ DDRB = 0x0f; USART_Init(); while(1){ USART_Transmit(0x03); if(receive_data){ PORTB = receive_data; receive_data = 0; } } } ISR(USART_RX_vect){ receive_data = UDR0; } |
| --- | --- |

| /* This is the same code as above but without interrupt Master Code */ void USART_Init() { UBRR0 = 0; DDRD ¦= 16; UCSR0C = (1<<UMSEL01)¦(1<<UMSEL00)¦(1<<UCPHA0)¦(1<<UCPOL0); UCSR0B = (1<<RXEN0)¦(1<<TXEN0); UBRR0 = 832; } volatile char USART_Tx_Rx(volatile char data){ while (!(UCSR0A & (1<<UDRE0))); /* Wait for empty transmit buffer */ UDR0 = data; while (!(UCSR0A & (1<<RXC0))); /* Wait for data to be received */ return UDR0; } void setup(){ DDRB = 0xff; USART_Init(); while(1){ PORTB = USART_Tx_Rx(0x05); } } | /* Slave Code */ void USART_Init(){ UBRR0 = 0; DDRD ¦= 16; UCSR0C = (1<<UMSEL01)¦(1<<UMSEL00)¦(1<<UCPHA0)¦(1<<UCPOL0); UCSR0B = (1<<RXEN0)¦(1<<TXEN0); UBRR0 = 832; } volatile char USART_Tx_Rx(volatile char data){ while (!(UCSR0A & (1<<UDRE0))); /* Wait for empty transmit buffer */ UDR0 = data; while (!(UCSR0A & (1<<RXC0))); /* Wait for data to be received */ return UDR0; } void setup(){ DDRB = 0xff; USART_Init(); while(1){ PORTB = USART_Tx_Rx(0x03); } } |
| --- | --- |

----------------------------- o ---------------------------